Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: POWER SYSTEM ANALYSIS							       	       Time: 3 hours

Subject Code: EE255		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Write the expression to find the per unit value of any electrical quantity.

2.	A generator rated at 25 MVA, 11KV has a reactance of 15%. Calculate it’s per unit reactance for a base of 40 MVA and 10KV.

3.	Define bus impedance matrix of a power system.

4.	What is the need for slack bus?

5.	Write down the relative frequency of occurrence of various types of faults.

6.	What are the causes of fault in power system?

7.	What are the constraint to be satisfied to solve load flow equations for a given load configuration?

8.	What is meant by acceleration factor in load flow solution?  What is its best value?

9.	Define swing curve. What is the use of swing curve?

10.	State the applications of equal area criterion.

PART – B (5 x 3 = 15 MARKS)

11.	What are the advantages of a p.u. quality?

12.	What is an impedance diagram? What are its applications?

13.	What are the reactances used in the analysis of symmetrical faults on the synchronous machines as its equivalent reactances.

14.	Compare Newton Raphson and fast decoupled method of load flow solution.

15.	List the various causes for voltage instability.

PART – C (5 x 15 = 75 MARKS)

16.	A 100 MVA, 33 KV, three phase generator has a reactance of 20 %. The generator is connected to motors through transmission lines and transformers. The motor has rated inputs of 30 MVA, 20 MVA and 50 MVA at 30 KV with 15 % sub transient reactance. The three phase transformers are rated at 110 MVA, 32 KV Δ/110 KV Y with leakage reactance of 8 %. The line has a reactance of 45. Selecting the generator rating as base quantities in generator circuit, determine base quantities in other parts of system and evaluate the p.u. values.

T2

T1

J45

M1

M2

M3

100MVA,

33KV,

(OR)

17.	Explain in detail the need for system analysis in planning and operation of power system.

[P.T.O]

18.	Explain in detail the formation of Ybus matrix using singular transformation.

(OR)

19.	Explain in detail the modeling of generator, load, shunt capacitor, transmission line and shunt reactor.

20.	Explain in detail the unbalanced fault analysis using Zbus matrix.

(OR)

21.	A synchronous generator rated 10 MVA, 6.6 KV hasXd''=0.15p.u. It’s negative and zero sequence reactances are 0.15 & 0.05 p.u respectively. The neutral of the generator is solidly grounded. The generator is operating at 6.6 KV without load and is disconnected from the system when a Double line to ground fault occurs at its terminals. Find the sub-transient current in the generator and the line-to-line voltages for sub-transient conditions.

22.	The load flow data for a 3-bus system is given in tables 1 and 2. The voltage magnitude at bus 2 is to be maintained at 1.0 p.u. The maximum and minimum reactive power limits for bus 2 are 0.3 and 0 p.u. respectively. Taking bus 1 as slack bus, determine the voltages of the various buses at the end of first iteration starting with a flat voltage profile for all buses except slack bus using N-R method.

Table 1: Line Impedance

| Bus code | Impedance | Line charging admittance Y’pq /2 |
| --- | --- | --- |
| 1-2 | 0.06+j0.18 | j0.05 |
| 1-3 | 0.02+j0.06 | j0.06 |
| 2-3 | 0.04+j0.12 | j0.05 |

Table 2: Assumed bus voltages, Generation and loads

| Bus code | Voltages p.u | Generation | Load |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | MW p.u | MVAR p.u | MW p.u | MVAR p.u |
| 1(Slack) | 1.06 | 0 | 0 | 0 | 0 |
| 2(PV) | 1 | 0.2 | 0 | 0 | 0 |
| 3 (PQ) | 1 | 0 | 0 | 0 | 0.25 |

(OR)

23.	Explain in detail with a flow chart, Gauss-Seidel Power Flow Analysis.

24.	Explain in detail the Swing equation and equal area criteria.

(OR)

Line

j0.15

G

T

Infinite bus

25.	Figure shows a generator supplying power to an infinite bus system. The reactances of various components of the system are marked on the figure. The generator is supplying a power of 80 MW with a terminal voltage of 1.09 p.u. A sustained fault occurs on the system and reduces the maximum power which the generator can supply to 0.5 p.u. Apply Runge-Kutta method. Inertia constant H of generator is 5 Mega Joules per MVA. Base MVA is 100.