Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: POWER SYSTEM PROTECTION AND SWITCHGEARS	       Time: 3 hours

Subject Code: EE256			       						             	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is the need for a protective scheme?

2.	What is symmetrical fault?

3.	On what principle do distance relays operate?

4.	What is the function of negative sequence relay?

5.	What is routine testing with regard to testing of circuit breakers?

6.	What is recovery voltage?

7.	What is the difference between surge diverter and surge absorber?

8.	What is switching surge?

9.	What are the drawbacks of an ungrounded system?

10.	Which type of grounding is suitable for radial lines? Why?

PART – B (5 x 3 = 15 MARKS)

11.	What is differential protection?

12.	What is static relay? Also list any three drawbacks of it.

13.	What are the merits of vacuum breakers?

14.	List out the harmful effects of lightning.

15.	What is arcing ground?

PART – C (5 x 15 = 75 MARKS)

16.	a.	Discuss the basic ideas of relay protection and also nature and causes of faults. 		(8)

b.	With a neat sketch, briefly explain the types of construction of a potential transformer. 																						(7)

(OR)

17.	a.	Explain in detail, the essential qualities of protection. 								(10)

b.	List out the consequences of faults. 												(5)

18.	Explain the principle and operation of an induction type directional over current relay with a neat diagram.

(OR)

19.	a.	Discuss in detail about the percentage differential protection employed for the protection of  	transformers. 																(10)

b.	A 3 transformer having line voltage ratio of 0.4KV/11KV is connected in star-delta and 	protective transformers on 400V side have a current ratio of 500/5. What must be the ratio 	of protective transformers on 11KV side? 										(5)

20.	a.	Discuss about resistance switching. 											(5)

b.	Explain capacitive current breaking with a suitable explanatory diagram. 			(10)

(OR)

[P.T.O]

21.	a.	Describe in detail, the construction and working of sulphur hexafluoride circuit breakers 	with a neat sketch. Also state its advantages. 									(10)

b.	In a short circuit test on a circuit breaker, the following readings were obtained on single 			frequency transient:

i.	Time to reach peak restriking voltage is 50 micro-seconds.

ii.	Peak restriking voltage is 100 KV.

Determine the average RRRV and frequency of oscillations.  						(5)

22.	a.	Explain how lightning is produced and the mechanism of lightning discharge. 		(10)                             b.	List out the undesirable effect caused by travelling waves to electrical equipment and how 	it can be prevented. 															(5)

(OR)

23.	Explain in detail about the construction and working of valve type surge diverter and Ferranti surge absorber with necessary sketches.

24.	Write short notes on the following:

a.	Resistance earthing. 															(8)

b.	Earthing transformers. 														(7)

(OR)

25.	What is insulation co-ordination? Discuss in detail, how insulation co-ordination is achieved among various components and apparatus present in a sub-station.