Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DIGITAL ELECTRONICS								      	       Time: 3 hours

Subject Code: EE257			       					               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What are the two types of logic levels used?

2.	What is a Hamming code?

3.	Define fan in.

4.	Which type of output circuit is used for wired logic operation in TTL?

5.	What is a multiplexer?

6.	What is a combinational logic circuit?

7.	What is a flip-flop?

8.	What is the advantage of state reduction in the design of sequential circuits?

9.	What is FPLA?

10.	Define PAL.

PART – B  (5 x 3 = 15 MARKS)

11.	Add 	a.	(923)8 and (67)8   		b.	(7F)16 and (BA)16.

12.	What happens if the output of a TTL gate with active pull-up gets connected to a +5V supply accidently?

13.	Differentiate implicant, prime-implicant and essential prime implicant.

14.	What is meant by race condition in an asynchronous sequential circuit?

15.	Draw the block diagram of a PLA device.

PART – C  (5 x 15 = 75 MARKS)

16. 	Make a K-Map for the function: f = AB+AC’+C+AD+AB’C+ABC. Express f in canonical SOP form. Also minimize it and realize the minimal expression using NAND gates only. 																					(3+4+8)

(OR)

17.	Explain the following with suitable examples: 	a.	Parity check codes   b.	 Hamming codes.																						(7+8)

18.	Explain the characteristics of digital ICs.

(OR)

19.	With neat sketch, describe about CMOS and NMOS logic gates.

20.	Minimize the following expression using a K-map with minimum number of NOR gates only.  F(A,B,C,D) = Σm(8,9,10,11,13,15,16,18,21,24,25,26,27,30,31).

(OR)

21.	Implement the expression using 	a.	one 16x1 multiplexer 		b. two 8x1 multiplexers. F(A,B,C,D)=Σm(0,2,3,6,8,9,12,14).	           											(7+8)

22.	Discuss and differentiate the operation of level and edge triggering of flip-flops.

(OR)

23.	Design a Mod-12 normal binary counter for the count sequence 0000 to 1011.

[P.T.O]

24.	Write a VHDL code for a full subtractor, using logic equations. Assume that the full subtractor has a 5-ns delay.

(OR)

25.	Discuss the concept of working and applications of the following memories:

a.	ROM	b.	PLA	c.	FPLS