Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: CONTROL SYSTEMS									      	       Time: 3 hours

Subject Code: EE259			       					               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is a signal flow graph?

2.	Define System

3.	Define Damping ratio.

4.	Define Pole and Zero.

5.	What is meant by phase margin?

6.	Define Cut-off frequency.

7.	What is a centroid?

8.	Write the characteristics equation of a closed loop system.

9.	Write the transfer function of PI-controller.

10.	What is a state variable model?

PART – B  (5 x 3 = 15 MARKS)

11.	Write the Mason’s gain formula.

12.	What is steady state error for unity and non-unity feedback systems?

13.	Mention the advantages of frequency response analysis.

14.	Using Routh criterion find the stability of the system represented by the characteristics

equation, S4+6S3+26S2+56S+80= 0.

15.	Draw the state model for a SISO linear system.

PART – C  (5 x 15 = 75 MARKS)

16.	Find the overall transfer function of the system whose signal flow graph is shown as below.

(OR)

17.	Determine the overall transfer function C(s)/R(s) for the system using block diagram reduction technique.

[P.T.O]

18.	A positional control system with velocity feedback is shown as below. What is the response of the system for unit step input?

(OR)

19.	A unity feedback system has G(s)=40(s+2)s (s+1)(s+4). Determine:

a.	type of the system				b.	Error co-efficient (Kp, Kv and Ka)

c.	Error for ramp input with magnitude 4.

20.	A unity feedback control system has an open loop transfer function G(s)=Ks (s2+4s+13)  Sketch the root locus.

(OR)

21.	a.	Briefly explain about Routh- Hurwitz criteria. 				          				(5)

b.	Determine the location of roots on the s-plane. The characteristics polynomial of the 				system is  S7+5S6+9S5+9S4+4S3+20S2+36S+36= 0. 			         			(10)

22.	A unity feedback control system hasG(s)=80s (s+2)(s+20). Sketch the Bode plot and determine gain margin, phase margin, gain cross over frequency and phase cross over frequency.

(OR)

23.	The open loop transfer function of a unity feedback system is given by G(s)=1s(1+s) (1+2s). Sketch the polar plot and determine the gain margin and phase margin.

24.	a.	Explain in detail about the concept of controllability and observability.    			(7)

b.	Obtain the state model of the given electrical network.		                     			(8)

(OR)

25.	a.	Examine the observability of the system given below.			          				(10)

x1x2x3 = 0100010−2−3x1x2x3+ 001u  and y = 341 x1x2x3

b.	Briefly explain about state variables and state model.								(5)