Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: COMPUTER ARCHITECTURE							       Time: 3 hours

Subject Code: EE272			       					               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Which is the basic component of an arithmetic circuit?

2.	What are the instruction code formats of a basic computer?

3.	What should be loaded as an initial value of stack pointer?

4.	The collection of all status bit conditions in the CPU is called _________.

5.	A floating point number that has a zero in the most significant position of the mantissa is said to have an _________.

6.	What are the methods used for division?

7.	What are the three possible errors checked during transmission by the asynchronous 	communication interface?

8.	The ROM portion of main memory is needed for storing an initial program called _________.

9.	Parallel processing can be challenges in four programmatic levels. What are they?

10.	The conventional Von Neumann machines are called _________.

PART – B  (5 x 3 = 15 MARKS)

11.	The following transfer statements specify a memory. Explain the memory operation in each case.

a.	R2        M[AR]

b.	M[AR]          R3

c.	R5          M [R5]

12.	What is auto increment / decrement mode of addressing?

13.	Draw the logic diagram of 2 bit x 2 bit array multiplier.

14.	What are the types of mapping procedures used in cache memory environment?

15.	What is an array processor?

PART – C  (5 x 15 = 75 MARKS)

16.	Design an 8 bit ALU. Concentrate on the design equations and circuit diagram of adders / subtractors and shifters. Analyze the performance using timing diagrams.

(OR)

17.	Draw the flowchart of an interrupt cycle and demonstrate the operation before and after an interrupt.

18.	a.	How do the RISC and CISC architectures differ in their characteristics?      			(7)

b.	Convert the following numerical arithmetic expression into reverse polish notation and show the stack operations for evaluation the numerical results.      					(8)

(3+4) [10(2+6) + 8]

(OR)

19.	Explain the ten different modes of addressing used in central processing unit. Give two example instructions for each mode.

[P.T.O]

20.	With a neat flow chart, discuss the operation of Booth’s multiplication algorithm and verify the result with a sample data set.

(OR)

21.	Draw the flowchart showing the division operation of floating point numbers and explain.

22.	What is I/O Processor? With a neat block diagram, explain the operation of 8089 IOP.

(OR)

23.	Derive the match logic of one cell of associative memory and write on its read and write operations.

24.	Analyze the balancing of subsystem bandwidth for a uni- processor environment.

(OR)

25.	What are the machine organizations listed by Flynn? With a neat block diagram, briefly discuss

their operations.