Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: POWER SYSTEM CONTROL							      	       Time: 3 hours

Subject Code: EE275			       					               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	The torque of an induction motor is proportional to the _____________.

2.	A surplus of Mega VARs tends to _____________ the voltage level of the system.

3.	For a constant setting of the speed changer the static increase in turbine power output is directly proportional to the _______________.

4.	Write the generator swing equation (GSE).

5.	What are the commonly used input signals for the Power System Stabilizer (PSS)?

6.	What are the classifications of excitation systems?

7.	The Energy Management System (EMS) communicates with generation, transmission and distribution systems through _____________ system.

8.	Write the expression for Area Control Error (ACE).

9.	Draw the incremental cost curve of steam unit.

10.	Write the generalized Lagrange function.

PART – B (5 x 3 = 15 MARKS)

11.	Mention the four reasons why the system frequency must not be allowed to deviate from a chosen constant value.

12.	Two generators are supplying power to a system. Their ratings are 50 and 500 MW respectively. The frequency is 60 Hz and each generator is half-loaded. The system load increases by 110 MW and as a result the frequency drops to 59.5 Hz. What must the individual regulations be if the two generators should increase their turbine powers in proportion to their ratings?

13.	What are the requirements of an excitation system?

14.	What are the functions of Energy Control Center?

15.	What is a base point and participation factor?

PART – C (5 x 15 = 75 MARKS)

16.	a.	Explain in detail various types of loads and their characteristics.  					(7)

b.	A 480 V, 3 phase induction motor powers a compressor, whose torque is assumed to be independent of speed. The motor runs at an initial speed corresponding to a per unit slip of s = 0.03. How will the motor load change if the voltage drops by one percentage? 	(8)

(OR)

17.	a.	Explain in detail the Mega VAR – Voltage (Q - |V|) interactions.					(7)

b.	All the generators and motors in a power system represent a total kinetic energy of 1500 MJ (or MW) as measured at rated frequency, 60 Hz. The system experiences a momentary power surplus at 5 MW. At what rate will the frequency increase?					(8)

18.	Draw the transfer function block diagram for a two area system provided with governor control and obtain the steady state frequency error following a step load change in both the areas.

(OR)

[P.T.O]

19.	What is meant by “Steady-State” instabilities? Explain it in detail.

20.	Describe in detail the modeling of excitation system components.

(OR)

21.	Discuss in detail the Static VAR Systems.

22.	Determine the state vectors, line flows and power injections at the buses using state estimation for the given measurements for the figure shown below.

Take 100 MVA base, X12=0.2, X13=0.4, X23=0.25 Without errors, measurements are M13=5MW and M32=40MW and Take 100 MVA base, X12=0.2, X13=0.4, X23=0.25. Without errors, measurements are M13=5MW and M32=40MW and with errors, measurements are M12=62MW, M13=6MW and M32=37MW.

(OR)

23.	With an example, explain the four operating states of a power system in the security 	perspective with an example.

24.	The fuel cost equation for three thermal plants in `/ hr are given by

F1=561+7.92P1+0.001562P12.F2=310+7.85P2+0.00194P22.F3=78+7.97P3+0.00482P32.

where P1, P2 and P3 are in MW. The total load is 850 MW. Neglect line losses and generating limits, find the optimal dispatch and the fuel cost in `/ hr.

(OR)

25.	Explain in detail the lambda iteration method for solving economic scheduling problem, without transmission loss.