Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MATERIAL SCIENCE									       Time: 3 hours

Subject Code: EE276			       					               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What is a crystal?

2.	What is basis?

3.	What is Doping?

4.	List some types of extrinsic semiconductors.

5.	Define Electric dipole.

6.	What do you mean by dielectric losses?

7.	Define magnetic susceptibility.

8.	Mention the types of magnetic material.

9.	What are metallic glasses?

10.	List some different forms of nano materials.

PART – B (5 x 3 = 15 MARKS)

11.	Define inter-atomic distance and inter planar distance.

12.	What is a Crystal Defect? List its various types.

13.	Write short notes on Hall Effect and its importance.

14.	State some limitations of intrinsic semiconductors.

15.	Write some mechanical properties of nano materials.

PART – C (5 x 15 = 75 MARKS)

16. 	What are Miller Indices? How will you determine the Miller indices of a given plane? Give its features.

(OR)

17.	a.	Obtain Miller Indices of a plane whose intercepts are 4, 4, 2 units along the three axis.																					(8)

b.	Lattice constant of a BCC crystal is 0.36nm. Find its atomic radius.         			(7)

18.	Discuss the variations of Fermi level with temperature in an intrinsic semiconductor.

(OR)

19.	a.	The intrinsic carrier density is 1.5*1016/m3. If the electron and hole mobilities are 0.13 and 	0.05m2 v-1s-1, calculate its electrical conductivity.                           					(7)

b.	Find the intrinsic resistivity of Ge at room temperature 300K if the carrier density is 2.15 * 	10 13 /cm3  and if μe and μh is 3900, 1900.                                           					(8)

20.	Explain the various Polarization mechanisms in dielectrics.

(OR)

21.	a.	A parallel plate capacitor consist of 2 plates each area of 5*10-4m2. They are separated by 	distance 1.5*10-3m and filled with dielectric of relative permittivity 6. Calculate the charge 	on the capacitor if it is connected to a 100V DC supply.  							(8)

[P.T.O]

b.	The dielectric constant of He gas at NTP is 1.0000684. Calculate the electronic polarisability of He atoms if the gas contain 2.7*10 25 atoms per m3.				(7)

22.	The Magnetic field strength of copper is 106 ampere/meter. If the magnetic susceptibility of copper is 08 * 10-5 calculate the Flux density of magnetization in copper.

(OR)

23.	Discuss the domain structure in ferromagnetic material in detail.

24.	What is Meissner Effect? Explain the general properties of superconducting material and its types.

(OR)

25.	What are the mechanical, electrical, chemical properties of metallic glasses? State the advantages of using metallic glasses as transformer core material.