Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ILLUMINATION ENGINEERING	       					       	       Time: 3 hours

Subject Code: EE293			       						              	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	The unit for luminous intensity is ____________.

2.	The light enters into the eye ball through ____________.

3.	The three types of fluorescent lamps are ____________, ____________ and ____________.

4.	____________ is used as the material for the filament of incandescent lamps.

5.	According to Lambert’s law, illumination on surface is given by the expression ____________.

6.	For measuring the illumination of surfaces, ____________ photometers are used.

7.	Space/Height ratio = ____________.

8.	For reading, generally illumination of ____________ lm/m2 is considered sufficient.

9.	A well designed lighting scheme is one which avoids ____________ and ____________.

10.	The equipments used in flood lighting are of ____________ type and ____________ type.

PART – B (5 x 3 = 15 MARKS)

11.	What are the four factors that affect illumination. Mention the type of photometers used to measure them.

12.	What are the advantages of a Filament lamp?

13.	Deduce the relation between the lux and the foot candle and between candle power and lumen.

14.	What are the factors that affect utilization factor?

15.	A room 8 m x 12 m is lighted by 15 lamps to a fairly uniform illumination of 100 lm/m2.  Calculate the utilization coefficient of the room given that the output of each lamp is 1600 lumens.

PART – C (5 x 15 = 75 MARKS)

16.	With the help of a neat diagram of a human eye, explain the parts and their functions.

(OR)

17.	Explain in detail about the method of measurement of luminous intensity and luminous flux.

18.	What are fluorescent lamps?  Explain its types.  Draw and explain the circuit for instant starting of a fluorescent lamp.

(OR)

19.	Discuss about various types of light fittings.

20.	a.	Discuss the laws governing illumination from point, line and surface sources.         	(10)

b.	A lamp of 100 candela is placed 1m below a plane mirror which reflects 90% of light
      		falling on it.  The lamp is hung 4m above ground.  Find the illumination at a point on the
      		ground 3m away from the point vertically below the lamp.                                      	(5)

(OR)

21.	What do you understand by polar curves as applied to a light source?

[P.T.O]

22.	What are all the factors to be considered for the design of a lighting scheme?  Write in detail about the lighting design procedure for factory lighting.

(OR)

23.	a.	A drawing hall 40m x 25m x 6m high is to be illuminated with metal filament gas filled
      lamps to an average illumination of 90 lm/m2 on a working plane 1 m above the floor. 
      Estimate suitable number, size and mounting height of lamps.  Sketch the spacing layout.
      Assume coefficient of utilization of 0.5, depreciation factor of 1.2 and spacing/height
      ratio of 1.2.

Size of lamps                                     200 W         300 W             500 W

Luminous efficiency (lm/W)                   16                18                    20    			(7)

b.	Write short notes on office lighting.												(8)

24.	a.	Explain in detail about the two design principles involved in street lighting.			(8)

b.	What are the requirements of good lighting?                                                           	(7)

(OR)

25.	a.	Write in detail about the design procedure for flood lighting.                                   	(10)

b.	Estimate the number of 1000 W floodlight projectors required to illuminate the upper 
      		75m of one face of a 96m tower of width 13m if approximate initial average luminance 
      		is to be 6.85 cd/m2.  The projectors are mounted at ground level 51 m from the base of the
      		tower.  Utilization factor is 0.2: reflection factor of wall is 25% and efficiency of each
      		lamp is 18 lm/W.																  (5)