Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: AUTOMOTIVE ELECTRONICS						      	      Time: 3 hours

Subject Code: EE294			       					               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The oxygen sensor measures the difference between the amount of oxygen in the _____________ and the amount of oxygen in __________.

2.	The wheel speed sensor is a type of _____________ used to measure speed of the wheel by its _____________.

3.	The modern starter motor is either a _____________ or a _____________ wound on a current electric motor with a starter solenoid mounted on it.

4.	The Starter switch can also be called by the name _____________.

5.	A carburetor is a device that blends _____________ and _____________ for an engine.

6.	In fuel injection systems, when signaled by the _____________ the fuel injector opens and sprays the _____________ fuel into the engine.

7.	Why is negative earthing  and not positive earthing provided on the vehicle?

8.	What is a trafficator?

9.	To make sure the engine has enough fuel during throttle opening, to make the engine accelerate smoothly, _____________ is used.

10.	What is the use of distributor less ignition?

PART – B  (5 x 3 = 15 MARKS)

11.	Describe the working principle of throttle flow sensors.

12.	What are the different starter driver units of a starter motor? Explain in detail.

13.	Write a note on multi point fuel injection systems and controls.

14.	How does an electrical fuel pump work? Explain with suitable diagrams.

15.	What do you mean by open loop and closed loop control systems? Define deceleration leaning and ideal speed control.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain various applications for following sensors in context with automotive systems.

a.	Position Sensor                                      c.	Velocity Sensor

b.	Altitude Sensor                                      d.	Torque Sensor

(OR)

17. 	With the aid of sketches explain the construction and working of solenoids and stepper motors as  actuators in vehicles.

18. 	Describe the principle and construction of a Starter motor.

(OR)

19. 	Give a detailed note on Starter switches.

20.	With the aid of a block diagram and flow charts explain the starting and acceleration enrichment of the Fuel Injection system of a petrol car having electronic engine management.

(OR)

[P.T.O]

21. 	Write about the different types of triggering devices in a contact less ignition system.

22. 	Explain head light dazzling in detail. What are all the methods followed to prevent it. Give a brief note on side light.

(OR)

23. 	Sketch and explain the working of fuel and temperature gauges in vehicles. Briefly write about the principle of wiper system’s operation in automobiles.

24. 	a.	Explain the concept of steering control system in detail.

b.	Give a note on distributor less ignition mechanism in an integrated engine control system.

(OR)

25.	Electrically controlled power steering system has lots of advantages. With the neat sketches, explain the working principle and two of the advantages of power steering system. Write a short note on 4-wheel steering system.