Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ELECTRICAL ENGINEERING							       Time: 3 hours

Subject Code: EE296								               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Three 10 k resistors are connected in series. A 20 k resistor is connected in parallel across

one of the 10 resistors. The voltage source is 24 V. The total current in the circuit is _______.

2.	A unity power factor in an ac circuit indicates that the circuit is ________.

3.	Maximum power is transferred when load resistance is equal to ________ resistance.

4.	The difference between the half-power frequencies is called ________.

5.	Two three-phase induction motors when connected across a 400 V, 50 Hz supply runs at 1440 	r.p.m. and 940 r.p.m. respectively. Which of the two motors runs at higher slip?

6.	A Transformer core is built from thin stampings to reduce ________ loss.

7.	Neon tubes are widely used for ________.

8.	A capacitor is connected across the fluorescent tube circuit in order to ________ of the tube circuit.

9.	Wattmeter is an instrument which is used to measure ________ of a electric circuit or an 	appliance which is connected to the supply in terms of Watts.

10.	The type of CRT used in CROs is ________.

PART – B  (5 x 3 = 15 MARKS)

11.	Find the currents I1, I2 and I3 in the network in the figure below.

I1

3A

2A

1A

2A

4A

I2

I3

4A

2A

1

3

4

2

12.	In an RLC series circuit, the applied source voltage is e(t) 	= 2 sin (10, 000t) V. The value of R is 10  and the value C is 0.1 F. What shout be the value of L so that the circuit would resonate at this frequency of the source? Find the quality factor and the bandwidth of this circuit.

13.	Enumerate the reason for skewing of a cage rotor of a three phase induction motor.

14.	State the requirements of good lighting.

15. 	What are the objectives of earthing?

[P.T.O]

PART – C  (5 x 15 = 75 MARKS)

16. 	a.	Find the voltage across the combination of resistors in the figure below. Also determine the 	current in each of them.														(7)

R1

100 

R2

100 

6 A

R3

200 

R4

300 

b.	Use mesh equation to find V0 in the network shown in the figure below.				(8)

2

12 V

+

4 A

I2

I3

I1

V0

2 

4

(OR)

17. 	a.	Find the total equivalent impedance of the network in the figure below and also find the 			polar form of the current in the rightmost capacitor.

10 mH

~

+



50 pF

50 pF

50 pF

7 sin (106 t) V

e (t)

+

10 mH

10 mH

↑

↓

2

4 A

8 A

4 

V1

V2

1

b.	Use nodal analysis to find all the branch currents in the circuit in the figure below.

18.	a.	Compute V0 in the network in the figure below using superposition.				(7)

6

4

4

8

36 V

48 V

+



V0

[P.T.O]

b.	The input to the primary of the circuit shown in the figure below, is 25 mV at a frequency 	of 1.1 MHz and the secondary is tuned too this frequency. Calculate:

i.	the value of C	ii.	the input impedance		iii.	the secondary current

iv.	the voltage across the capacitor.											(8)

~

+



I1

I2

13 

13 

22H

196H

25  103 ∠00 V

28H

+

C

(OR)

19. 	a.	Use Thevenin’s Theroem of find V0 in the circuit shown in the figure below.			(10)

6

4

6 

12 

20 V

V0

2 

8 

↑

4 

4 



+

+



2 A

b.	A series resonant circuit has a resonance frequency of 500 kHz and a bandwidth of 25 kHz.

What is its quality factor? Determine the exact and approximate values of its 3-db cutoff frequencies. If the value of the capacitor in this circuit is 0.1 F, find the values of R and L. 																			(5)

20. 	a.	Explain with neat sketches, the principle of operation of a three phase induction Motor.

(10)

b.	Write short notes on AC and DC Servo Motors.									(5)

(OR)

21. 	a.	With neat sketches, explain the principle of operation of a DC motor.				(10)

b.	Write short notes on Universal Motors.											(5)

22.	a.	With a neat sketch, explain the working of a fluorescent lamp.						(10)

b.	Write short notes on quality of lighting.											(5)

(OR)

23. 	a.	With a neat sketch, explain the working of an incandescent lamp.					(8)

b.	Write short notes on LED-LCD displays.										(7)

24.	With neat sketches, explain:

a.	Domestic wiring 	 			b.	Staircase wiring

(OR)

25. 	 Write short notes on:

a.	CRO						b.	UPS