# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: DIGITAL CONTROL SYSTEMS						      	       Time: 3 hours

# Subject Code: EI214						       		      		       	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	The process of converting a discrete time continuous valued signal into a discrete time discrete discrete valued signal is called ______________.

2.	The transfer function of ZOH  is______________.

3.	The bilinear transformation is performed by substituting ______________ in the characteristics equation.

4.	The transfer function of an LDS system is called ______________.

5.	What is the state transition matrix of discrete time system?

6.	Define controllability.

7.	Write the transfer function of a digital PID controller.

8.	What are the problems posed by the 8 and 16 bit microprocessors to control system applications?

9.	The Hybrid stepper motor combines the features of ______________.

10.	Define resolution.

PART – B  (5 x 3 = 15 MARKS)

11.	State Shannon’s sampling theorem.

12.	Define z- transform of a unit step signal.

13.	Give the steps involved in determining the pulse transfer function of G(z) from G(s).

14.	What is meant by digital quantization?

15.	What is the step angle of a four phase stepper motor with 12 stator teeth and 3 rotor teeth?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Explain typical sampled-data control system in detail.                                             	(7)

b.	With a neat sketch, explain the reconstruction of sampled signals using hold circuits.	(8)

(OR)

17.	a.	With neat diagram explain the successive approximation technique of analog to digital

conversion.                                                                                                                  	(8)

b.	Explain the binary weighted resistor technique of digital to analog conversion.        	(7)

18.	A sampled data control system of order one with transporation lag is shown in the figure.	Determine the condition for system stability if δ>T.

[P.T.O]

(OR)

19.	For the sampled-data system of the figure, find the response to unit step input, where

G(s)=1(s+1).

20.	Obtain three different canonical state variable models corresponding to the transfer function:

G(z)=4z3−12z2+13z−7(z−1)2(z−2).

(OR)

21.	Determine the state controllability and observability of the given system.

X(k+1)=01−1−2x(k)+01u(k)c(k)=11x(k)

22.	Discuss the implementation of digital control algorithms using microprocessors.

(OR)

23.	a.	Discuss the functional block diagram of microcontrollers.                                          	(8)

b.	Write a note on ‘Dead beat algorithm’. 											(7)

24.	Discuss the various modes of excitation in VR stepper motor.

(OR)

25.	With neat sketches, describe the hardware features of the design of a microprocessor based 	temperature control system.