Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ULTRASONIC INSTRUMENTATION					       	       Time: 3 hours

Subject Code: EI229				       					              	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Due to the limitations of middle ear, the upper frequency limit in a human is approximately _________ Hz.

2. 	__________ is a cyclic pressure with a frequency greater than the upper limit of human hearing.

3. 	Magnetostrictive effect is also known as ________ effect.

4. 	Material which transduces magnetic energy to mechanical energy and vice versa is said to be

____________ .

5. 	_____________ of wave disturbances through a medium known as velocity.

6.	The _____________ of pulse width to pulse period is known as duty cycle.

7. 	A small magnitude of pressure disturbances are propagated through a fluid at a definite

_________ relative to the fluid.

8. 	When blood flows in the ____________ of energy transmission, the transit time is shortened.

9. 	___________  is used to measure slight movement of body due to forces exerted by breathing 	of heart and pumping of blood.

10. 	Well ________ the audible range, ultrasonic nebulizer produces high-intensity sound energy.

PART – B (5 x 3 = 15 MARKS)

11. 	Define Acoustics.

12.	What is Ultra sound?

13. 	Define a Flaw.

14.	Write a note on Resolution.

15. 	What is an Ecorotor?

PART – C (5 x 15 = 75 MARKS)

16.	Discuss in detail about the modes of sound wave propagation.

(OR)

17. 	Arrive at an equation of a plane progressive wave.

18.	Draw a neat electronic circuit diagram to produce ultrasonic wave based on magneto-strictive effect.

(OR)

19.	How will you determine velocity of an ultrasonic wave experimentally? Describe with a neat sketch.

20.	Ultrasonic pulser-receivers are well suited to general-purpose ultrasonic testing. Discuss in detail.

(OR)

[P.T.O]

21.	How will you detect flaws in materials? Draw an ultrasonic flaw detector circuit also.

22.	Write short notes on the following:

a.	Ultrasonic drilling.			b.	Ultrasonic soldering.

(OR)

23.  Give a detailed note on the following:

a.	Cold welding.				b.	Ultrasonic cleaning and drying.

24.	Discuss in detail about Medical sonography.

(OR)

25.	Explain about B-scan mode.