Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: LOGIC AND DISTRIBUTED CONTROL SYSTEMS		      Time: 3 hours

Subject Code: EI236			       						                    	      Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Write down the full form of SCADA and DDC.

2.	Write down the equation of a PID Controller.

3.	Mention any two Input and Output Modules of PLC.

4.	What are the basic building blocks of SCADA?

5.	List out the alternative programming languages of Ladder Logic.

6.	What is the purpose of PID function in PLC?

7.	Write down the abbreviation of LCU and what is the functionality of LCU in DCS?

8.	What does the term Data Highway refer to in DCS?

9.	List out three Industrial Communication Standards.

10.	Name different fields of applications of DCS in industries.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the concept of Linearization with a neat diagram and circuit as well.

12.	Develop a Ladder Programming for Flashing the Traffic Light for 10 secs Interval.

ON – 10 secs; OFF – 10 secs.

13.	Explain the function of sequencer in PLC.

14.	What are the functions and requirements of Field Control Units?

15.	Write a note on DCS System integration with PLC.

PART – C  (5 x 15 = 75 MARKS)

16.	With a clear block diagram distinguish between DAS and SCADA.

(OR)

17.	A temperature between 100oC and 300oC is converted into a 0 to 5 V signal. This signal is fed to an 8 bit ADC with a 5V reference. What is the actual measurement range of the system?

a. 	What is the resolution?

b. 	What Hex output results from 169 oC?

c. 	What temperature does a hex output of C5H represent?

18.	a. 	Develop a Ladder Program for running a conveyor till it fills a container with 10 	Sealed Bottles.  																(7.5)

b. 	Draw the Ladder Program for automating the Car Park Area and display a light for “House Full” if the parking has cars of 50. Assume you could count both incoming and outgoing cars. 																		(7.5)

(OR)

19.	Vividly explain the following functions of the PLC with example ladder logic:

a. 	Timer - ON Timer.															(7.5)

b. 	Counter Function - UP Counter.												(7.5)

20.	a. 	Differentiate between SKIP and MCR functions of PLC with example programming.	(10)

b. 	List out the Arithmetic functions and Number Comparison functions in PLC. 		(5)

[P.T.O]

(OR)

21.	Develop the ladder logic for a Dishwasher function using a PLC. List out the sequences and I/O devices.

22.	What is DCS? Draw its architecture and explain the function of each in detail.

(OR)

23.	a.	"Evolution of DCS is a remarkable milestone in Automation" - Justify this statement.

(7.5)

b. 	What is a Data Highway in DCS? Explain about it clearly. 						(7.5)

24.	Write a detailed note on system integration of PLC and DCS.

(OR)

25.	What is the need of communication in DCS? Explain the standards adopted in the communication protocol.