Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: PROCESS DYNAMICS AND CONTROL				      	       Time: 3 hours

Subject Code: EI246			       						              	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State principle of conservation of fundamental quantity.

2.	Which of the following system will give an inverse response?

a.	G(s) = 100.1 s +1 − 50.04 s+1		b.	G (s) = 100.2 s+1 −50.3s+1

3.	How is derivative kick avoided in a PID based control loop?

4.	What is proportional band?

5.	Process reaction curve method is a _________ loop tuning method.

6.	In a carburetor to mix air with gasoline vapor _________ control action is preferred.

7.	What is valve sizing?

8.	What is the importance of a valve positioner?

9.	Give 2 examples of domestic heat exchanger applications.

10.	Differentiate fire tube and water tube boilers

PART – B  (5 x 3 = 15 MARKS)

11.	While analyzing a process, the degree of freedom was found to be +2. What do you infer from this?

12.	Discuss the cause and remedy for ‘Reset windup’ in a PID control action.

13.	Define the merits of a cascade control system.

14.	Differentiate fail to open and fail to close type control valves.

15.	Explain the task of a reflux drum in a distillation column setup.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the following with the help of proper examples.

a.	Batch process and Continuous process.		b.	System with inverse response.

c.	System with self regulation.

(OR)

17.	For the process shown in the figure below, level h3 is stable at 10 cm when inlets Q1 and Q2 are 100 mm3/sec and 120 mm3/sec. When the inlets are increased to 120 mm3/sec and 140 mm3/sec suddenly, what will be the new stable value of h3? ( A1=A2 = A3 = 1250 mm2, R1=2 sec/ mm2, R2 = 1 sec/mm2 and R3 = 2 sec/mm2).

[P.T.O]

18.	a.	The differential gap in an ON-OFF controller is ±1% of set point. When the set point is 300°C, the 	steam flow is ON for 2 mins and OFF for 3 mins. If the set point is changed to 250°C, what will be 	the new period of oscillation of the controller output? Plot the new controller output.

b.	Analyze the impact of the following controllers on a first order system in a closed loop control 			system. 		i.	P – alone, 	ii.	I – alone, 	iii.	D – alone

(OR)

19.	For the given error pattern plot the graph of P and P+I+D controller output from 0 to 7 sec. The controller parameters are : Kp=5, Ki = 2sec-1, Kd = 3sec and mo=60%.

20.	A 4th order system can be approximated as 2e−1.5s1+8s using process reaction curve method. For this system tune the controller in following modes: P mode, P+I mode and P+I+D mode.

(OR)

21.	With necessary derivations explain the necessity and working of the following control strategies:

a.	Feed forward control mode.		b.	Feed forward with feedback control mode.

c.	Inferential control.

22.	a.	How does flashing in a flow control valve? What is the impact of it?

b.	Classify control valves based on flow-lift characteristics. How are valve plugs designed to achieve 		these characteristics?

(OR)

23.	With neat schematic diagrams explain any three types of control valve actuators.

24.	a.	Draw the schematic of a binary distillation process and analyze the degrees of freedom.

b.	Implement feed forward control for controlling a boiler drum parameters. Show the schematic 			using P&I diagram standards.

(OR)

25.	Present a case study on implementing control actions for the following cases:

a.	Bottom product in a Distillation column.

b.	Heat exchanger.