Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: SENSORS AND TRANSDUCERS						      	       Time: 3 hours

Subject Code: EI247				       				      		       	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Instruments are briefly classified as ________ instruments and __________ instruments.

2.	What are the dynamic characteristics of measurement systems?

3.	Give two applications of Pirani gauge.

4.	The Bourdon tube is called as ________ transducer and LVDT is called as ________ transducer.

5.	Give the principle of a Strain gauge.

6.	Define Gauge factor.

7.	What is the principle of a Capacitive transducer?

8.	Give examples for natural and synthetic group of piezo-electric materials.

9.	What are the applications of Hall-effect transducer?

10.	Mention the classifications of Encoders.

PART – B  (5 x 3 = 15 MARKS)

11.	Distinguish null and deflection type instruments.

12.	Write a short note on inverse transducer.

13.	What is meant by Synchros?

14.	A capacitive transducer uses two quartz diaphragms of area 750 mm2 separated by a distance of 3.5 mm. A pressure of 900 kN/m2 when applied to the top diaphragm produces a deflection of 0.6 mm. The capacitance is 750 pF when no pressure is applied to the diaphragms. Find the value of capacitance after the application of a pressure of 900 kN/m2.

15.	Draw the schematic diagram of a Seismic accelerometer.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Briefly explain the elements of generalized measurement systems.			        	(7)

b.	List and explain any four main static characteristics of measurement systems.		(8)

(OR)

17.	Explain in detail the types of errors.

18. 	a.	Discuss the factors influencing the choice of transducers. 				 			(10)

b	Distinguish analog and digital transducers.										 (5)

(OR)

19. 	Describe briefly about the transfer characteristics of transducers.

20. 	a.	What is meant by Rosettes? Discuss their forms.									(8)

b.	Describe Resistance-Temperature characteristics of thermistors.					(7)

(OR)

21. 	a.	How is temperature measured using a thermocouple?								(8)

b.	Explain the operation of L.V.D.T.												(7)

[P.T.O]

22. 	Explain how capacitive transducers are used for measurement of linear displacement using

a.	overlapping area of plates.														(8)

b.	distance between the two plates.												(7)

(OR)

23. 	Explain the principle, mode of operation and properties of piezo-electric crystals.

24. 	a.	Explain how a Hall-effect transducer is used to measure current.					(7)

b.	Discuss the working of a Shaft encoder.											(8)

(OR)

25. 	Write short notes on:

a. 	Optical encoder. 																(8)

b. 	Smart sensors.	 															(7)