Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: SIGNAL CONDITIONING CIRCUITS					      	       Time: 3 hours

Subject Code: EI258						       		        	      	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define input offset current.

2.	The average rate of change of input offset voltage per unit change in temperature is called ______________.

3.	What is the resulting output for a practical differentiator, whose input is a square wave?

4.	Draw the circuit of voltage follower.

5.	What is the key characteristic of a Butterworth filter?

6.	Which one of the following is having high common mode rejection ratio?

a. integrator					b. Inverting amplifier

c. Instrumentation amplifier	d. non-inverting amplifier

7.	What is meant by ripple rejection in a voltage regulator?

8.	Give the pin details of 7800 series of voltage regulators.

9.	List the various digital phase detectors available.

10.	List any two practical applications of Phase Locked Loops.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the circuit diagram of IC 741 of an inverting amplifier with a feedback resistance of 100kΩ, and find the maximum output offset voltage caused by the input offset current.

12.	Draw the circuit diagram of voltage to current convertor with grounded load. Brief the working of the circuit.

13.	Design a Butterworth low-pass filter at a cutoff frequency of 1 kHz with a passband gain of 2. 	Draw the circuit diagram.

14.	Draw the schematic circuit diagram of Motorola’s MC1723 general purpose regulator.

15.	Explain the application of PLL as a Frequency Shift Keying demodulator.

PART – C  (5 x 15 = 75 MARKS)

16.	a. 	What is meant by slew rate in operation amplifier? 								(3)

b. 	What are the causes of slew rate? 												(5)

c. 	Derive a relation for slew rate.													(7)

(OR)

17.	a. 	Derive the expression for the output voltage vo for the circuit shown in the figure below.																						(8)

[P.T.O]

b.	Design a differential amplifier with variable gain. Discuss the advantages of it.		(7)

18.	a. 	An inverting amplifier is used as a summer to sum three different signals coming from 	three different sensors. The output of the amplifier is 1.49 V, when the input potential of 	first two sensors is given as 120 mV and 450 mV, and they are connected with an input 	resistance of 10 kΩ and 12 kΩ respectively. The third sensor input is connected with an 	input resistance of 5 kΩ. Find the input voltage from the third sensor when the amplifier 	has a feedback resistance of 22 kΩ.												(7)

b.  	Draw the circuit with all the calculated values for the above given problem.			(4)

c. 	Draw the positive clipper circuit and explain the working.							(4)

(OR)

19.	Draw the circuit diagram of a 555 timer configured as monostable multivibrator, and explain the operation.

20.	a. 	Design a second order low pass filter at a high cut off frequency of 1kHz. Draw the circuit 	diagram of the filter.															(10)

b. 	Draw the frequency response of the designed filter.								(5)

(OR)

21.	Design a signal conditioning circuit using instrumentation amplifier for strain gauge, which will measure the change in strain as change in resistance. Discuss the significant factors of instrumentation amplifier over other differential amplifiers.

22.	a. 	Explain the working of a high voltage 723 general purpose regulator with a neat circuit 	diagram.																		(10)

b. 	List and explain the characteristics of three terminal IC regulators.					(5)

(OR)

23.	a. 	Draw the functional diagram of 723 general purpose regulator, and explain various current 	limiting features of 723 regulator.												(12)
b. 	Discuss the application of voltage regulators.									(3)

24.	a.	What is mean by Phase Locked Loops (PLL)? Explain the working of PLL with a neat 	block diagram.  																(10)

b. 	What are the advantages and disadvantages of monolithic PLLs over discrete PLLs?	(5)

(OR)

25. 	a.	Explain Exclusive OR phase detector and Edge-triggered phase detector.			(12)

b. 	Compare the performance of both the phase detectors.							(3)