Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DIGITAL CONTROL SYSTEMS						       	       Time: 3 hours

Subject Code: EI259			       					               	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Conversion of a continuous time function to a sequence is called _____________.

2.	The highest frequency that can be represented by a digital signal is _____________ radians.

3.	Mention any two methods of discretization.

4.	The transformation _____________ maps the s plane into z plane.

5.	_____________,  _____________ and _____________ are the methods of realization of pulse transfer functions.

6.	Controllability and obervability concepts are _____________ concepts.

7.	In bilinear transformation s = _____________.

8.	According to Jury’s stability test, the necessary conditions to be satisfied for the stability of the system with characteristic polynomial F(z) are_____________.

9.	The transfer function of a PID controller is given by _____________.

10.	Incremental form of PID algorithm is also called as _____________ form of PID algorithm.

PART – B (5 x 3 = 15 MARKS)

11.	Write down the advantages of sampled data control systems.

12.	How is the s plane mapped into z plane?

13.	Define controllability and observability.

14.	What is the stability criterion for sampled data control system?

15.	What are the problems attributed to time delays in control applications?

PART – C (5 x 15 = 75 MARKS)

16. 	a.	Discuss the structure of a multivariable sampled data control system.				(7)                                b.	Explain the analysis of sampling process in frequency domain.    					(8)

(OR)

17.	a.	Explain how are the sampled signals reconstructed using hold circuits. 				(7)

b.	With the help of a circuit diagram, discuss three bit D/A converter in detail. 			(8)

18.	a.	Find the one sided z-transform of the discrete sequence generated by mathematically 	sampling the following continuous time function, eat sinωt. 							(7)

b.	Determine the inverse z-transform of the following z-domain function:

F(z) = (3z2 + 2z +1) / (z2 +3z +2).                                                                                 	(8)

(OR)

19.	a.	Solve the difference equation c(k+2) +3c(k+1) +2c(k) = u(k).  Given that c(0) = 1;         	c(1) = 3; c(k) =0 for k < 0.													(8)

b.	Find C(z) / R(z) for the following closed loop sampled data control system where          Go(s) = (1esT) / s.															(7)

[P.T.O]

20.	a.	Check whether the following system is controllable and observable X = Ax +bu; Y=cx

where

A = 010016.31060000001−1.0637000			b =0−1.445800.9639		c = 1000.			(8)

b.	A feedback system has closed loop transfer function Y(s)/R(s)=[10 (s +4)]/[s(s+1)(s+3)]. Construct three different state models for this system:

i.	One where the system matrix A is a diagonal matrix.

ii.	One where A is in first companion form.

iii.	One where A is in second companion form.                                     				(7)

(OR)

21.	Discuss in detail about the realization of pulse transfer functions.

22.	a.	Check for stability of the sampled data control system represented by the following 	characteristic equation: z3 – 0.2z2 – 0.25z + 0.05 = 0. 								(7)

b.	Explain how stability analysis is done using bilinear transformation.                          	(8)

(OR)

23.	Explain signal quantization and coefficient quantization methods in detail.

24.	a.	With a block diagram, explain the description of microcontrollers.                              (8)

b.	A PID controller is described by the following relation between input e(t) and output u(t).

u(t) = Kc[e(t) + (1/TI) ∫ e(t) dt + TD+ de(t)/d(t)].  Using the trapezoidal rule for integration and backward difference approximation for the derivatives, obtain the difference equation model of the PID algorithm.  Also obtain U(z) / E(z).                   	 				(7)

(OR)

25.	Write short notes on the following:

a.	Position PID algorithm

b.	Incremental PID algorithm.