Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ELECTRICAL MEASUREMENTS AND INSTRUMENTS	       Time: 3 hours

Subject Code: EI260						       		        		      	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define calibration.

2.	True or false: Moving iron voltmeter can be used for both DC and AC voltage measurements.

3. 	In an induction type meter, maximum torque is produced when the phase angle between the two fluxes is __________ degrees.

4. 	The other name of voltage coil of an electrodynamometer type wattmeter is __________.

5. 	True or false: In the student type potentiometers, it is impossible to obtain true zero.

6. 	The other name of voltage ratio box is __________ divider.

7. 	Low resistance values are in the order of __________ ohm and below.

8. 	In a Bridge at balance condition, the current through the detector is __________.

9. 	Write down the expression for ‘Q’ of a coil.

10. 	True or false: Vibration galvanometers are not used at power frequencies.

PART – B  (5 x 3 = 15 MARKS)

11.	Define sensitivity of a voltmeter.

12.	List the type of errors that occur in a wattmeter.

13.	List the applications of AC potentiometers.

14.	Give the detectors used for DC and AC measurements.

15.	List the methods to measure unknown inductance.

PART – C  (5 x 15 = 75 MARKS)

16. 	Explain the construction and working principle of D’Arsonaval galvanometer.

(OR)

17. 	Discuss about the rectifier type instrument and how it is used as ammeters and voltmeters.

18. 	Explain briefly about the errors occurring in electrodynamometer instruments.

(OR)

19. 	Explain the working principle of an induction type Watt-hour meter.

20. 	Discuss the applications of DC potentiometers with neat sketches.

(OR)

21. 	What do you mean by potential transformers? Give the equations and phasor diagram.

22. 	Discuss briefly about Kelvin’s double bridge for the measurement of an unknown resistance.

(OR)

23. 	Describe briefly about Megger.

24.  Discuss briefly how the Schering’s bridge can be used for the measurement of capacitance.

(OR)

25.	Explain how the Wien’s bridge can be used for the measurement of frequency.