Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: SIGNALS AND SYSTEMS									       Time: 3 hours

Subject Code: IT240			       					             			       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Give an example of a finite energy signal.

2.	Determine whether the system y[n]=x[n] is causal or not.

3.	State the conditions for causality and stability of a system with impulse response h[n].

4.	Find if the system y(t) = y(t1) + 2t y(t2) is time variant or not.

5.	State the frequency shifting property of a Discrete Time Fourier Transform.

6.	Give the general form of a discrete-time non-recursive filter.

7.	State the time shifting property of a Laplace Transform.

8.	What is meant by Nyquist interval?

9.	When does the ROC of a z-transform form a ring in the z-plane centered about origin?

10.	Find the z-transform of the impulse signal δ(nm).

PART – B (5 x 3 = 15 MARKS)

11.	Determine the fundamental period of the signal e(jΠ4)n.

12.	Check whether the given system y(t) = x(t) cos (t+1) is causal and stable.

13.	Deterrmine the Discrete Time Fourier Transform of the sequence x[n]={1, 1,1, 1}

14.	State and prove the convolution property of Laplace transform.

15.	Find the inverse z-transform of the signal  X(z)=11−az−1 ROC: |z| > |a| by long division method.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Find and sketch the even and odd components of the signal x(t)=t2−t  0≤t≤11≤t≤2 	(6)

b.	Determine the values of power and energy of the signal x[n] = ej(Π2n+Π4)  	(6)

c.	Find the fundamental period of the signal x[n] = e(j6Πn)						(3)

(OR)

17.	Prove the following:

a.	The power of the energy signal is zero over infinite time.							(5)

b. 	The energy of the power signal is infinite over infinite time.						(5)

c. 	x[n]=e2nu[n]is neither power nor energy signal.									(5)

18.	Convolve the two sequences x[n] and h[n] given by x[n] = αn u[n] ;  0 < α <1 and h[n] = u[n].

(OR)

19.	a.	Determine the convolution sum of two sequences x[n] = {3,2,1,2} and h[n] ={1,2,1,2 } 																						(10)

[P.T.O]

b. 	Determine the impulse response of the system characterized by the difference equation 	y[n] – 0.5 y[n–1] = x[n].														(5)

20.	Obtain the Fourier transform of a rectangular pulse of duration T and amplitude A as shown in the figure below and plot its magnitude and phase spectra.

(OR)

21.	Illustrate the characteristics and trade-offs in a continuous time system with the interpretation of an automobile suspension system as a low pass filter.

22.	Explain in detail how a signal can be reconstructed from its samples using interpolation. Assume the signal has been sampled using Nyquist criteria. Also comment on the effects of undersampling.

(OR)

23.	a. 	Determine the Laplace transform and ROC of the signal, x(t) = – eat u(–t).			(8)

b. 	State and prove the time scaling property of Laplace transform.					(7)

24.	a. 	Determine the causal signal x[n] having the z-transform X(z)=1(1−2z−1)(1−z−1)2.  (10)

b. 	Determine the inverse z-transform of the signal X(z)=11−3z−1+2z−2.				(5)

(OR)

25.	Determine the unit step response of the system whose difference equation is:

y[n] – 0.7y[n–1] + 0.12y[n–2] = x[n–1] + x[n–2] if y[–1] = y[–2] =1.