Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: HEAT ENGINES AND FLUID MACHINERY	              	              Time: 3 hours

Subject Code: ME223						       		        		       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define specific weight.

2.	Define the term pump.

3.	Define unit speed.

4.	Define impulse turbine.

5.	Define heat engine.

6.	What is a steady flow process?

7.	What type of cycle is used to run in the Spark Ignition engines?

8.	Define mechanical efficiency.

9.	What are the three basic modes in which heat is transferred?

10.	Define the emissivity of a grey body.

PART – B  (5 x 3 = 15 MARKS)

11.	Define compressibility.

12.	What are the effects of cavitations?

13.	Define Kelvin Plank’s statement.

14.	Differentiate using any two points between two stroke and four stroke engines.

15.	Define the term fin efficiency.

PART – C  (5 x 15 = 75 MARKS)

16.  A centrifugal pump is running at 1000 r.p.m. The outlet vane angle of the impeller is 45˚ and 	velocity of flow at outlet is 2.5 m/sec. The discharge through the pump is 200 litres/sec when 	the pump is working against a total head of 20 m. If the manometric efficiency of the pump is 	80%, determine: 	a.	the outside diameter of the Impeller		b.	the width of the impeller at 	outlet.

(OR)

17.	Explain with a neat sketch the working principle of a reciprocating pump.

18.	What is a draft tube? Why is it used in a reaction turbine? Describe with neat sketches different types of draft tubes.

(OR)

19.	A Pelton wheel is to be designed for a head of 60 m when running at 200 r.p.m. The Pelton wheel develops 95.6475 kW shaft power. The velocity of the buckets = 0.45 times the velocity of the jet, overall efficiency = 0.85 and co-efficient of velocity is equal to 0.98.

20.	5 kg of oxygen is heated in a reversible non-flow constant volume process from temperature of 50˚C until the pressure is doubled. Determine:	 a.	Final temperature		b.	work done	  c.	change in internal energy	d.	heat transferred	e.	change in enthalpy	f.	change in entropy. Assume Cv =  0.653 kJ/kg-˚K, Cp = 0.913 kJ/kg-˚K

(OR)

[P.T.O]

21.	A mass of 50 kg/min enters the control volume of a steady flow system at 2 bar and 100˚C and an elevation of 100 m above the datum. The same mass leaves the control volume at 150 m elevation at a pressure of 10 bar and temperature of 300˚C. The entrance velocity is 2400 m/min and exit velocity is 1200m/min. During the process 50,000 kJ/hour of heat are transferred to the control volume and the rise in enthalpy is 8kJ/kg. Calculate the power developed.

22.	Explain briefly Otto cycle with help of p-v and T-S diagram, and derive an expression for the ideal efficiency of Otto cycle.

(OR)

23.	The following observations were made during a test on a two-stroke cycle oil engine:

Room temperature = 20˚C

Bore = 20 cm

Stroke = 25 cm

Speed = 350 r.p.m.

Brake drum diameter = 1.2 m

Net brake load = 450 N

Mean effective pressure = 2.8 bar

Oil consumption = 3.6 kg/hr

Calorific value of oil = 41800 kJ/kg

Quantity of jacket cooling water = 455 kg/hr

Rise in temperature of jacket water = 28˚C

Temperature of exhaust gas entering the exhaust gas calorimeter = 320˚C

Temperature of exhaust gas leaving the exhaust gas calorimeter = 220˚C

Quantity of water passing through exhaust gas calorimeter = 8kg/min

Temperature rise of calorimeter water = 8˚C.

Determine the indicated and brake power, mechanical efficiency and brake thermal efficiency. Draw up a heat balance on one minute basis

24.	A steam pipe of 10 cm outside diameter is covered with two layers of insulating materials each of 2.5 cm thick, one having thermal conductivity thrice the other. Show that the effective conductivity of two layers is less when better insulating material is inside than when it is on the outside. Determine the percentage reduction.

(OR)

25.	A long steel rod, 2 cm in diameter, is to be heated from 427˚C to 538˚C. It is placed concentrically in a long cylindrical furnace which has an inside diameter of 16 cm. The inner surface of the furnace is at a temperature of 1093˚C, and has an emissivity of 0.85. If the surface of the rod has an emissivity of 0.6, find the time required for the heating operation. Take the steel, ρ = 7845 kg/m3 and C = 0.67 kJ/kg-˚K.