Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ENGINEERING THERMODYNAMICS					       Time: 3 hours

Subject Code: ME228				       				        	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	State the concept of continuum.

2.	Define quasi static process.

3.	Write the significance of entropy.

4.	Define availability.

5.	Give examples for pure substances.

6.	What is meant by wet steam?

7.	When can a gas be considered as an ideal gas?

8.	State Avagadro’s law.

9.	What is meant by an air standard cycle?

10.	Give the significance of Mean Effective Pressure.

PART – B  (5 x 3 = 15 MARKS)

11.	Write the equations of first law of thermodynamics applicable to a closed and open system.

12.	Define COP of a refrigerator and heat pump.

13.	Find the enthalpy of 1 kg of wet steam at 1 MN/m2 and 0.8 dry.

14.	Differentiate between HCV and LCV.

15.	Compare Otto, Dual and Diesel cycles for same compression ratio and heat rejection.

PART – C  (5 x 15 = 75 MARKS)

16.	0.15 kg of air having temperature and pressure 800 K and 600 kN/m2 respectively is expanded polytropically to a pressure of 200 kN/m2. If the index of expansion is 1.3, calculate:

a.	 amount if heat transfer 	b.	change in internal energy 	c.	work done

(OR)

17.	A nozzle receives 1200 kg/h of steam at 1.3 MN/m2, specific volume 142 liters/kg, internal energy 4400 kJ/kg and negligible speed. At the exit, the pressure is 100 kN/m2, specific volume 1600 liters/kg and internal energy 2200 kJ/kg. Calculate:		a.	the exit velocity of the steam	b.	If all the kinetic energy were later converted into work, what would be the power?

18.	Two reversible heat engines operate on Carnot cycle. They work in series between a maximum and minimum temperature of 550C and 20C. If the engines have equal thermal efficiencies and the first rejects 450 kJ to the second, calculate:

a.	Temperature at which heat is supplied to the second engine.

b.	Heat taken from the source.

c.	Work done by each engine.

(OR)

19.	a.	A heat pump is required to deliver 250 kJ/s to a high temperature heat reservoir at a 	temperature of 200C. It takes in heat from a heat sink at a temperature of 350C and 	requires 31 kW power for its operation. Calculate the change of entropy of the heat pump 	and the heat reservoir combined together and show that this arrangement is possible. 	Consider the processes as irreversible.											(8)

[P.T.O]

b.	Prove that during isothermal process, Δs=Rlnv2v1								(7)

20.	Calculate the enthalpy, internal energy, specific volume and entropy of 1 kg of steam at 10 	bar and 350C.

(OR)

21.	Steam at 17 bar and 0.95 dry is throttled to 13 bar and then expanded 	adiabatically to 0.3 bar. 	Find the changes in internal energy and entropy per kg of steam in each process.

22.	Write the Vander Waals equation and derive the expression for its constants.

(OR)

23.	Explain the method of determination of calorific value of fuel using a Bomb calorimeter.

24.	An Otto cycle having a compression ratio of 11 takes in air at 1 bar and 25C. The maximum temperature of the cycle is 155C. Calculate 		a.	air standard cycle 	b.	pressures and temperatures at each characteristic points 	c.	mean effective pressure of the cycle.

(OR)

25.	A Brayton cycle has intake conditions of 1 bar and 300 K, maximum temperature of 1200 K and maximum pressure of 6 bar. Find 	a.	air standard efficiency 	b.	turbine exhaust temperature 		c.	temperature at the exit of the compressor.