Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: THERMAL ENGINEERING						       		       Time: 3 hours

Subject Code: ME229				       				              	              Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Classify the IC engines on the basis of number of working stroke.

2.	Which test is used for finding frictional power of a multi cylinder engine?

3.	Why should there be a convergent and divergent type of steam nozzle for a thermal power 	plant?

4.	What is the significance of an Impulse turbine?

5.	What is the effect of clearance volume in a reciprocating air compressor?

6.	Write the reason for employing multi stage air compressor.

7.	What is sub cooling?

8.	Write the advantages of vapour absorption and vapour compression refrigeration system.

9.	What are the industrial applications of psychrometry?

10.	Define SHF.

PART – B (5 x 3 = 15 MARKS)

11.	Compare the 2 stroke and 4 stroke engines based on construction.

12.	What is the difference between impulse and reaction turbine?

13.	Explain the limitation of maximum pressure in a reciprocating compressor.

14.	What are the merits of vapour absorption refrigeration system over vapour compression refrigeration system?

15.	What is the adiabatic saturation process?

PART – C (5 x 15 = 75 MARKS)

16.	Explain in detail about the ignition systems of an internal combustion engines.

(OR)

17.	The following observations were made during the test on a single cylinder, 4 stroke gas engine:

Bore = 300 mm; Stroke = 400 mm; RPM = 200; No. of explosion per minute = 88; Gas used=11.72 m3/h; pressure of gas = 175 mm of Hg above atmospheric pressure; Barometric pressure = 755 mm of Hg; mean effective pressure = 6 bar; calorific value of gas used = 21500 kJ/m3 at NTP; Net load on the brake = 1839.75 N; Brake drum diameter = 1 m; ambient temperature = 20ºC. Calculate:

a. 	Mechanical efficiency. 	b.	Brake thermal efficiency.

18.	A group of convergent Divergent nozzles is required to pass 9 kg of steam per second. The pressure of supply being 5.6 bar absolute and the back pressure is 1.4 bar absolute. The steam is initially dry saturated and the expansion is assumed to be in equilibrium. The loss by friction in the divergent part is 12 % of the total adiabatic enthalpy drop. Find for complete expansion, the total areas of cross section required at the throats and exits of the group.

(OR)

19.	With the help of neat sketches and velocity diagram, explain different types of compounding of turbines.

[P.T.O]

20.	A two cylinder single acting air compressor is to deliver 15 kg of air per minute at 6.5 bar from suction conditions 1 bar and 13 ºC. Clearance may be taken as 4 % of the stroke volume and the index for both compression and re-expansion as 1.3. Compressor is directly coupled to a four cylinder four stroke petrol engine which runs at 1800 rpm with brake mean effective pressure of 6 bar. Assuming a stroke bore ratio of 1.1:1 for both engine and compressor and a mechanical efficiency of 55% for the compressor, calculate the required cylinder dimensions.

(OR)

21.	Derive the expression for volumetric efficiency of a reciprocating compressor in terms of  pressure ratio.

22.	A refrigerating plant works between the temperature limits of 268 K in the evaporator and 298 K in the condenser. The working fluid ammonia has a dryness fraction of 0.6 at entry to the compressor. If the machine has a relative efficiency of 50%, calculate the amount of ice formed during a period of 24 hours. The ice is to be formed at 0 ºC from water at 20 ºC and 6 kg of ammonia is circulated per minute. Specific heat of water is 4.186 kJ / kg K and latent heat of ice is 335 kJ /kg. The table of properties of ammonia is given below:

| Temp | Specific Enthalpy kJ/kg |  | Entropy of liquid kJ/kg K, sf |
| --- | --- | --- | --- |
|  | Sat.liquid | Sat. vapour |  |
| 298 | 298.9 | 1167.1 | 1.124 |
| 268 | 158.2 | 1280.8 | 0.630 |

(OR)

23.	With neat sketch, explain the working of practical ammonia – water, vapour absorption refrigeration system.

24.	A restaurant is to be maintained at 27 ºC dry bulb temperature and 21ºC wet bulb temperature. It has a sensible load of 2100 kJ/min and latent heat load of 840 kJ/min. The outside air at 38 ºC dry bulb temperature and 27ºC wet bulb temperature is supplied at the rate of 20 m3 per minute directly into the room through ventilators and by infiltration. The outside air to be conditioned is passed through a cooling coil which has an apparatus dew point temperature of 15 ºC; 60% of the total air is recirculated from the conditioned space and mixed with conditioned air after the cooling coil. Find:

a.	the required condition of air after the cooling coil before mixing with recirculated air.

b.	condition of air entering the restaurant.

c.	mass of fresh air entering the cooling coil.

d.	By-pass factor for the cooling coil.

e.	refrigerating load of the cooling coil.

(OR)

25.	Explain in detail about the classification of Air-conditioning system in different aspects.