Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: HEAT AND MASS TRANSFER							       Time: 3 hours

Subject Code: ME230						       		               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is heat flux?

2. 	List any two applications of heat transfer study.

3. 	Define ‘Geometric factor (GF)’

4. 	When are the Heisbler charts used?

5. 	Give two examples for fins.

6. 	What is Nusselt number (Nu)?

7. 	What are the types of heat exchangers?

8. 	List out two examples for both heat transfer and mass transfer.

9. 	Define ‘Emissivity’.

10. 	What is meant Kirchoff’s identity?

PART – B  (5 x 3 = 15 MARKS)

11. 	What is the basic law of heat conduction?

12. 	What do you understand from film temperature?

13. 	Explain mass transfer by convection.

14. 	Define Fick’s law of diffusion.

15. 	Briefly discus about radiation shields.

PART – C  (5 x 15 = 75 MARKS)

16. 	Derive an expression for temperature distribution under one dimensional steady state heat    conduction and without heat generation for a hollow cylinder or tube.

(OR)

17. 	The wall of building is a composite consisting of 250 mm layer of common brick  (k=0.72 W/m K) and 10 mm layers of Gypsum plaster (k=0.12 W/m K) on both the sides of the bricks. During a hot day (at steady state), the temperature of outside plaster (exposed to ambient air) is 40°C and the temperature of inside plaster(exposed to inside air) is 25°C. Find:

a.	the heat flow rate through the wall per unit area of the wall	b.	 the temperature of the 	interface of brick and outside plaster.

18.	a.	Find the expression of cooling of a lumped body.									(7)

b.	Define:	a.	 Biot number (Bi) 	b.	 Fourier number (Fo).						(8)

(OR)

19. 	A solid steel ball 5 cm in diameter and initially at 450° C is quenched in  a controlled environment with convective coefficient of 115 W/m2 K. Determine the time taken by centre to reach a temperature of 150 °C. Take Cp =420 J/Kg K,  = 8000 Kg/m3 and k=46 W/m K.

20. 	a.	Explain the concept of velocity boundary layer development over a flat plate.		(8)

b.	Write the correlation for heat transfer from sphere for turbulent flow.				(7)

(OR)

[P.T.O]

21. 	Water at 10 °C is to be heated to 40° C in a tube of 2 cm diameter. The outside of the tube is 	wrapped with an electric heating element that produces a uniform flux of 200W/m3 over the 	surface and mass flow rate is 0.2Kg/s. Find:

a.	The Reynolds number 	b.	The heat transfer coefficient 	c.	Length of the pipe

The relevant properties of water at 25 C are given below-

Ρ = 997 kg/m3		Cp = 4180 J/kg K    k= 0.608 W/mK	     µ= 910 x 10-6 Poise

22. 	Water at the rate of 1 kg/s is heated from 35°C to 75°C by oil in a double pipe counter-flow heat exchanger. Oil has a specific heat of 1.9 kJ/kg k and it enters the heat exchanger at 110°C and 2.5 kg/s. The overall heat exchanger coefficient is 300 W/m2K. Calculate the area of heat exchanger tube.

(OR)

23. 	A container has binary mixture of oxygen and carbon dioxide gases at 21°C and 1.52  bar. The mean velocity of oxygen is 0.08 m/s and carbon dioxide is  0.02m/s. Also molar fraction of oxygen is 0.4. Find   a.	Molar fraction of carbon dioxide  b. Molecular weight of mixture.

c.	molar concentration of oxygen and carbon dioxide.	d.	Mass concentration of both 	gases.

24. 	a.	Two large parallel plates having emissivity of 0.5 each exist at temperature of 1200 K and   	300 K.	 i.	Find the heat transfer rate per m2 of the plate. 	ii. 	Find heat transfer rate 	if a 	radiation shield with an emissivity of 0.5 on both sides is placed between the two 	plates. 																		(10)

b.	Explain the types of reflection of radiation.										(5)

(OR)

25. 	What is greenhouse effect? Why is it a matter of great concern among environmental scientists?