Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DESIGN OF MECHANICAL TRANSMISSION SYSTEMS	       Time: 3 hours

Subject Code: ME231				       					       	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	For long distance drive, ______________ is employed.

(a) V-belt		(b) Flat belt		(c) Round belt		(d) Timing belt

2.	Which number is useful in the design of a Journal bearing?

(a) Reynolds Number (Re)			(b) Nusselt Number (Nu)

(c) Prandtl Number (Pr)			(d) Sommerfield Number (S)

3.	The most preferable profile for the gear is _____________.

(a) involute profile		(b) cycloidal profile		(c) trocoidal profile	(d) spiral profile

4.	Which of the following rope will be most flexible?

(a) 6 x 7		(b) 6 x 19		(c) 6 x 37		(d) 8 x 19

5.	In straight bevel gear drive, the axes are	:

(a)	non-intersecting and the teeth are straight and tapered.

(b) 	non-intersecting and the teeth are straight and non-tapered.

(c) 	intersecting and the teeth are straight and tapered.

(d) 	intersecting and the teeth are straight and non-tapered.

6.	In which gear drive, is self locking available?

7.	The cylindrical gear drive which is used to transmit power between non-parallel and non-intersecting shafts is known as _____________.

8.	The number of gears employed in a gear box is kept to the minimum by arranging the speeds of the spindle in _____________.

(a) Arithmetic Progression		(b) Geometric Progression

(c) Polynomial Progression	(d) Logarithmic Progression

9.	The contact between the cam and the follower is a line contact - State True or False.

10.	The smallest circle drawn to the pitch curve from the centre of rotation of the cam is known as _____________.

PART – B (5 x 3 = 15 MARKS)

11.	Distinguish between rolling contact and sliding contact bearings.

12.	What is a herringbone gear? Where is it used?

13.	Give a brief account for skew gears.

14.	What does the ray diagram of a gear box indicate?

15.	Mention some applications of Geneva mechanism.

PART – C (5 x 15 = 75 MARKS)

16.	Design a V-belt drive to transmit 50 kW at 1440 rpm from an electric motor to a textile machine running 24 hours a day. The speed of the machine shaft is 480 rpm.

(OR)

17.	Design a Journal bearing for a centrifugal pump for the following specifications:

Diameter of the journal = 75 mm.

Speed of the journal = 1140 rpm.

Load on each journal = 11500 N.

[P.T.O]

18.	A 10 kW, 1440 rpm motor drives a line shaft at 300 rpm by a chain drive. The centre to centre distance of the shaft is to be approximately 0.5 m. The motor shaft diameter is 40 mm. The starting torque of the motor is 2.5 times the running torque. The load applied is with moderate shock. Select a suitable roller chain drive.

(OR)

19.	A pair of helical gears subjected to moderate shock loading is to transmit 30 kW at 1500 rpm of the pinion. The speed reduction ratio is 4 and the helix angle is 20°. The service is continuous and the teeth are 20° FD in the normal plane. For the gear life of 10,000 hours, design the gear drive.

20.	Design a bevel gear drive to transmit 3.5 kW with the following specifications:

Speed ratio = 4.

Driving shaft speed = 200 rpm.

Material for pinion: Steel.

Material for wheel: Cast iron.

Life = 25,000 hours.

(OR)

21.	Design a worm gear drive to transmit a power of 22.5 kW. The worm speed is 1440 rpm and the speed of the wheel is 60 rpm. The drive should have a minimum efficiency of 80 % and above. Select suitable materials for the worm and wheel and decide upon the dimensions of the drive.

22.	Sketch the arrangement of a six speed gear box. The minimum and maximum speeds required are around 450 and 1400 rpm. Drive speed is 1440 rpm. Construct the speed diagram of the gear box. Use standard output speeds and standard step ratio. Also calculate the number of teeth in each gear.

(OR)

23.	Draw the ray diagram and kinematic layout of a 12 speed gear box for a milling machine having an output of speeds ranging from 180 to 2000 rpm. Power is applied to the gear box from a 6 kW induction motor at 1440 rpm. Use standard step ratio.

24.	Design a Ratchet and Pawl mechanism for the following data:

Torque = 500 Nm.

Number of teeth = 18.

Material for ratchet: Steel.

Material for pawl: Steel (hardened).

(OR)

25.	Design a screw jack to lift a load of 50 kN through 200 mm. The screw is made of C45 steel and the nut is made of Cast-iron grade 20. Calculate the effort required to lift the load at a leverage of 500 mm.