Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MECHANICS OF MACHINES - I						       	       Time: 3 hours

Subject Code: ME254				       					              	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Attachment of a car mirror represents a ____________ type of pair.

(a) sliding	(b) turning 	(c) rolling	(d) spherical

2.	Cycle chain is an example of kinematic chain – State True or False.

3.	In a friction clutch, the intensity of pressure is maximum at ___________.

(a) outer radius	(b) inner radius	(c) anywhere else

4.	The fore end of a ship is called___________.

(a) bow		(b) aft		(c) port		(d) star-board

5.	In a radial cam, the follower moves in a direction__________ to the axis.

6.	For high speed engines, the cam follower should move with _________.

(a) uniform motion	(b) simple harmonic motion	 (c) cycloidal motion

(d) uniform acceleration and retardation

7.	The product of circular pitch and diametral pitch is equal to __________.

(a) π		(b) 2π		(c) 3π		(d) 4π

8.	The path of contact in an involute gear is a straight line – State True or False.

9.	For isochronous governors, which condition is true?

(a) h1 > h2	(b) h1 < h2	(c) h1 = h2	(d) h1 = 2h2

10.	The coefficient of sensitiveness of a governor is the ratio between _________.

(a) range of speed and maximum speed	(b) range of speed and minimum speed

(c) range of speed and mean speed		(d) maximum speed and minimum speed

PART – B (5 x 3 = 15 MARKS)

11.	Distinguish between a machine and a structure.

12.	What are the functions of a clutch?

13.	Define undercutting in a cam. How does it occur?

14.	State the Law of Gearing.

15.	Define the terms: 		a.	Sensitiveness and 	b.	Hunting of governors.

PART – C (5 x 15 = 75 MARKS)

16.	a.	What is meant by inversion of a chain? Sketch and explain any three inversions of a four 		bar chain.																	(9)	 b.	In a four bar chain ABCD, AD is fixed and is 120 mm long. The crank AB is 30 mm long 	and rotates at 100 rpm clockwise. While the link CD = 60 mm oscillates about D, BC and 	AD are of equal length. Find the angular velocity of link CD when angle BAD = 60°.	(6)

(OR)

17.	In a slider crank mechanism, the length of crank OB and the connecting rod AB are 125 mm and 500 mm respectively. The centre of gravity of the connecting rod is 250 mm from the slider A. The crank speed is 600 rpm clockwise. When the crank has turned 45° from the inner dead centre position, determine		a.	linear velocity and acceleration of the midpoint of the connecting rod 	b.	angular velocity and angular acceleration of the connecting rod.

[P.T.O]

18.	A multi plate clutch transmits 55 KW of power at 1800 rpm. Coefficient of friction for the friction surface is 0.1. Axial intensity of pressure is not to exceed 160 KN/m2. The internal radius is 80 mm and is 0.7 times the external radius. Find the number of plates needed to transmit the required torque.

(OR)

19.	a.	Explain the terms steering, pitching and rolling in a naval ship.						(6)

b.	Each road wheel of a motor cycle has a mass moment of inertia of 1.5 kg-m2. The rotating 	parts of the engine of the motor cycle have a mass moment of inertia of 0.25 kg-m2. The 	speed of the engine is 5 times the speed of the wheels and is in the same sense. The mass 	of the motor cycle with its rider is 250 kg and its centre of gravity is 0.6 m above the 	ground level. Find the angle of wheel if the motor cycle is travelling at 50 km/hr and is 	taking a 	turn of 30 m radius. The wheel diameter is 0.6 m.						(9)

20.	Draw the profile of a cam operating with a knife edged follower having a lift of 30 mm. The cam raises the follower with Simple Harmonic Motion for 150° of its rotation followed by a period of dwell for 60°. The follower descends for the next 100° rotation of the cam with uniform velocity motion again followed by a dwell period. The cam rotates at a uniform velocity of 120 rpm and has a least radius of 20 mm. What will be the maximum velocity and acceleration of the follower during the lift?

(OR)

21.	A cam rotating clockwise with a uniform speed is to give the roller follower of 20 mm diameter with the following motion:

Follower to move outwards through a distance of 30 mm during 120° of cam rotation.

Follower to dwell for 60° of cam rotation.

Follower to return to its initial position during 90° of cam rotation.

Follower to dwell for the remaining 90° of cam rotation.

The minimum radius of the cam is 45 mm and the displacement of the follower is to take place with simple harmonic motion on both the outward and return strokes. Draw the cam profile when the line of stroke of the follower is offset by 15 mm from the axis of the cam.

22.	a.	Derive an expression for the length of arc of contact in a pair of meshed spur gears. 	(8)	 b.	A pinion having 20 teeth engages with an internal gear having 80 teeth. If the gears have 	involute profiled teeth with 20° pressure angle, module 10 mm and addendum of 10 mm, 	find the path of contact, arc of contact and contact ratio.							(7)

(OR)

23.	In an epicyclic gear train, an arm carries two gears A and B having 42 and 58 teeth respectively. If the arm rotates at 215 rpm in the anticlockwise direction about the centre of the gear A which is fixed, determine the speed of gear B. If the gear A instead of being fixed, makes 365 rpm in the clockwise direction, what will be the speed of gear B?

24.	The lengths of the upper and lower arms of a Porter governor are 200 mm and 250 mm respectively. Both the arms are pivoted on the axis of rotation. The central load is 150 N, the weight of each ball is 20 N and the friction of the sleeve together with the resistance of the operating gear is equivalent to a force of 30 N at the sleeve. If the limiting inclinations of the upper arms to the vertical are 30° at minimum position and 40° at maximum position, taking friction into account, find the range of speed of the governor.

(OR)

25.	a.	A Proell governor has all the four arms of length 250 mm. The upper and lower ends of the 	arms are pivoted on the axis of rotation of the governor. The extension arms of the lower 	links are each 100 mm long and parallel to the axis when the radius of the ball path is     	150 mm. The mass of each ball is 4.5 kg and the mass of the central load is 36 kg. 	Determine the equilibrium speed of the governor.									(9)

b.	Can a Porter governor be isochronous? Justify.	            							(6)