Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: MECHANICS OF MACHINES - II							       Time: 3 hours

Subject Code: ME255			       					             		       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is a free body diagram?

2.	Define inertia force.

3.	What is the function of a flywheel?

4.	Define the term co-efficient of fluctuation of energy.

5.	Define dynamic balancing.

6.	What is a balancing machine?

7.	Name the basic elements of a vibration.

8.	Distinguish between free and forced vibration.

9.	Define torsional vibration.

10.	What is torsional equivalent shaft?

PART – B (5 x 3 = 15 MARKS)

11.	State D’Alembert’s principle and its applications.

12.	Explain the turning moment diagram of a four-stroke I.C engine.

13.	State the conditions for complete balancing of rotating masses.

14.	Distinguish between longitudinal, transverse and torsional vibration.

15.	Determine the ratio of amplitudes of rotors of torsional vibrations of a two rotor system.

PART – C (5 x 15 = 75 MARKS)

16.	A four bar chain mechanism ABCD is shown in figure. Calculate the value of torque required  and all the constraint forces on links for static equilibrium of the mechanism, if force F=2000 N in the direction shown. The dimensions of linkages are as follows: AB=200 mm; BC=370 mm; CD=250 mm; AD=215 mm and CE=100 mm.

(OR)

17.	Derive an expression for the inertia force in the reciprocating parts, neglecting the weight of the connecting rod.

[P.T.O]

18.	a.	Define co-efficient of fluctuation of speed and deduce suitable expression.			(5)

b.	The maximum and minimum speed of a flywheel are 242rpm and 238rpm respectively. The mass of flywheel is 2600 kg and radius of gyration is 1.8 m. Find mean speed of flywheel, maximum fluctuation of energy and co-efficient of fluctuation of speed.	(10)

(OR)

19.	The turning moment diagram for a multi-cylinder engine has been drawn to a scale of          1mm = 325 Nm vertically and 1 mm = 3° horizontally. The areas above and below the mean torque line are 26, +378, 256, +306, 302, +244, 380, +261 and 225 mm2. The engine is running at a mean speed of 600 rpm. The total fluctuation of speed is not to exceed ±1.8% of the mean speed. If the radius of flywheel is 0.7 m, find the weight of flywheel.

20.	Explain the direct and reverse cranks method of balancing and deduce an expression for the primary and secondary unbalance in a reciprocating engine.

(OR)

21.	A shaft carries four rotating masses A, B, C and D which are completely balanced. The masses B, C and D are 50 kg, 80 kg, and 70 kg respectively. The masses C and D make angles of 90° and 195° respectively with mass B in the same sense. The masses A, B, C and D are concentrated at radius 75 mm, 100 mm, 50 mm and 90 mm respectively. The plane of rotation of masses B and C are 250 mm apart. Determine: 	a.	the mass A and its angular position

b.	the positions of planes of A and D.

22.	a.	What are the causes and effects of vibration? 									(5)

b.	Define the terms vibration isolation and transmissibility. 							(10)

(OR)

23.	In a single degree damped vibrating system, a suspended mass of 3.75 kg makes 12 oscillations in 7 seconds when disturbed from its equilibrium position. The amplitude of vibration reduces to 0.33 of its initial value after four oscillations. Determine:

a.	stiffness of the spring 		b.	logarithmic decrement	c.	damping factor

d.	the damping co-efficient.

24.	Three rotors A, B and C having moment of inertia of 2000, 6000 and 3500 kg-m2 respectively are carried on a uniform shaft of 0.35 m diameter. The length of the shaft between the rotors A and B is 6 m and between B and C is 32 m. Find the natural frequency of the torsional vibrations. The modulus of rigidity for the shaft material is 80 GN/m2.

(OR)

25.	A reciprocating I.C engine is coupled to a centrifugal pump through a pair of gears. The shaft from the flywheel of the engine to the gear wheel has a 48 mm diameter and is 800 mm long. The shaft from the pinion to the pump has 32 mm diameter and is 280 mm long. The pump speed is four times the engine speed. Moments of inertia of the flywheel, gear wheel, pinion and pump impeller are 1000 kg-m2, 14 kg-m2, 5 kg-m2 and 18 kg-m2 respectively. Find the natural frequency of the torsional oscillations of the system. Take, G= 80 GN/m2.