Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: DESIGN OF MACHINE ELEMENTS					      	       Time: 3 hours

Subject Code: ME257			       					               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is impact load?

2.	Stress concentration in loading is more serious in__________.

3.	What are the different types of loads that can act on machine components?

4.	The yield point in static loading is___________ as compared to fatigue loading.

5.	_________ and _________ is considered to design a shaft.

6.	________ material is used for flange coupling.

7.	A rivet is specified by, shank dia

8.	A cotter joint is used to transmits__________.

9.	The length of piston usually varies between, D and 1.5D

10.	Bending stress induced in flywheel arms due to __________.torque

PART – B  (5 x 3 = 15 MARKS)

11.	What is meant by design stress or working stress?

12.	Define spring rate and spring index.

13.	Differentiate joint and coupling.

14.	Give some examples for permanent and temporary joints.

15.	Define the following terms:

a.	Fluctuation of speed

b.	Co-efficient of fluctuation speed

c.	Stresses in flywheel rim

PART – C  (5 x 15 = 75 MARKS)

16.	What are the factors to be considered for the selection of materials for the design of machine elements? Discuss.

(OR)

17.	A mild steel shaft of 50 mm diameter is subjected to a bending moment of 2000 N-m and a torque T. if the yield stress of the steel in tension is 200 MPa, find the maximum value of this torque without causing yielding of the shaft according  to 		a. The maximum principal stress;	b.	 the maximum shear stress; 	c. The maximum distortion energy theory of yielding.

18.	Explain how the factor of safety is determined under steady and varying loading by different methods.

(OR)

19.	A railway wagon weighing 50 kN and moving with a speed of 8 km per hour has to be stopped by four buffer springs in which the maximum compression allowed is 220 mm. Find the number of turns in each spring of mean diameter 150 mm. The diameter of spring wire is 25 mm. Take G = 84 kN/mm2.

[P.T.O]

20.	A line shaft ABCD, 9 m long, has four pulleys A, B, C and D at equal distance apart. Power of 45 kW is being supplied to the shaft through the pulley C while the power is being taken off equally from the pulleys A, B and D. The shaft runs at 630 r.p.m. Calculate the most economical diameters for the various portions of the shaft so that the shear stress does not exceed 55 MPa. If the shear modulus is 85 GPa, determine the twist of the pulley D with respect to the pulley A.

(OR)

21.	A marine type flange coupling is used to transmit 3.75 MW at 150 r.p.m. The allowable shear stress in the shaft and bolts may be taken as 50 MPa. Determine the shaft diameter and the diameter of bolts.

22.	A double riveted lap joint with chain riveting is to be made for joining two plates 10 mm thick. The allowable stresses are; tensile stress = 60 MPa; Shear stress =50 MPa; compressive stress = 80 MPa. Find the rivet diameter, pitch of rivets and distance between rows of rivets. Also find the efficiency of the joint.

(OR)

23.	A knuckle joint is required to withstand a tensile load of 25 kN. Design the joint if the permissible stresses are:

σt = 56 MPa, τ = 40 MPa and σc = 70 MPa.

24.	Design a cast iron piston for a single acting four stroke engine for the following data:

Cylinder bore =100 mm; Stroke = 125 mm; Maximum gas pressure = 5 N/mm2; Indicated mean effective pressure = 0.75 N/mm2; Mechanical efficiency = 80%; Fuel consumption = 0.15 kg per brake power per hour; Higher calorific value of fuel = 42×103 kJ/kg; Speed = 2000 r.p.m.

(OR)

25.	A single cylinder, single acting, four stroke oil engine develops 20 kW at 300 r.p.m. The work done by the gases during the expansion stroke is 2.3 times the work done on the gases during the compression and the work done during the suction and exhaust strokes is negligible. The speed is to be maintained within ± 1%. Determine the mass moment of inertia of the flywheel.