Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: COMPUTATIONAL FLUID DYNAMICS		      		      	       Time: 3 hours

Subject Code: ME265				       				        	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	  is an elliptic equation. True or False?

2. 	Write down the energy equation for a 2D laminar fluid flow.

3. 	Write down the Taylor series expansion.

4. 	What is explicit method?

5. 	What is meant by transient analysis?

6. 	Write down central difference for .

7. 	Write down one dimensional convection-diffusion equation.

8. 	List any two schemes for discretization of convection term.

9. 	What is the use of under relaxation factor in SIMPLE procedure?

10. 	The eddy viscosity is also called as turbulent viscosity. True or False?

PART – B  (5 x 3 = 15 MARKS)

11. 	Explain the conditions for elliptic, parablolic and hyperbolic equations.

12. 	Discretize the following unsteady heat conduction equation using finite difference method.

13. 	What is the condition to impose no-slip and adiabatic wall boundary conditions?

14. 	Write short notes on upwind scheme.

15. 	What are the characteristics of a turbulence flow?

PART – C  (5 x 15 = 75 MARKS)

16. 	Give a detailed comparison between the merits and limitations of computational fluid dynamics and experimental fluid dynamics analysis.

(OR)

17. 	Discuss the transport equation for turbulent kinetic energy ‘k’ used in turbulent fluid flow.

18. 	Discuss the four basic rules obeyed by finite volume discretization equations.

(OR)

19.	Explain alternating direction implicit method.

[P.T.O]

20. 	Consider the problem of source-free heat conduction in an insulated rod whose length is 0.5 m and ends are maintained at constant temperatures of 100 0C and 500 0C respectively. The one-dimensional problem is governed by:

Thermal conductivity k equals 1000 W/(mK), cross-sectional area  is 10 x 10-3 m2. Obtain the set of equations (matrix form) that yield the steady state temperature distribution by dividing the length of the rod into five equal control volumes. (DO NOT SOLVE THE EQUATIONS).

(OR)

21.	Using finite volume method, obtain the equation in the form  for one-dimensional heat conduction in cylinder.

22.	Discuss the significance of three properties of discretisation schemes.

(OR)

23. 	Obtain the general discretization equation in the form  for the one dimensional convection diffusion equation using the central- difference formulation.

24. 	Explain, by drawing a flow chart, the procedure required to solve a two dimensional steady state flow problem using SIMPLE.

(OR)

25. 	Show how the two-dimensional continuity equation can be converted into an equation for pressure correction.