Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: FINITE ELEMENT ANALYSIS							       Time: 3 hours

Subject Code: ME269			       					               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is CST?

2.	What is LST?

3.	Define: shape function.

4.	Write the stiffness matrix equation for 2-D Problems in FEM.

5.	What is an isoparametric element?

6.	What is sub parametric element?

7.	Write any one property of shape function.

8.	Write any two shape function equations for CST in natural coordinate system.

9.	Write any two shape function equations for a four node rectangular element in natural coordinate system.

10.	What is a 2-D Pascal triangle?

PART – B  (5 x 3 = 15 MARKS)

11.	What is aspect ratio?

12.	What are the h and p methods of finite element method?

13.	Differentiate static and dynamic analysis

14.	What are the basic steps involved in FEM?

15.	Discuss potential energy functional for structural problems.

PART – C  (5 x 15 = 75 MARKS)

16.	Solve the following differential equation using Galerkin’s method:

y''+y=−x for (0 x 1) using the boundary condition y(0) = y(1) = 0.

(OR)

17.	Drive the functional for the following systems:

a.	A bar of length L and variable cross sectional area A is under a concentrated axial load P at its free end. The axial displacement at any point along the bar is governed by the following differential equation AE(d2u/dx2)=0, where E is the modulus of elasticity of the bar material.																		(7)

b.	A cantilever beam of length “L” and cross sectional moment of inertia “I” has a transverse load “P” at its free end. The lateral deflection “y” is given by EI(d4y/dx4)=0, where E is the modulus of elasticity of the bar material .										(8)

[P.T.O]

18.	Find the nodal displacements and element stresses for the tapered bar for the following cases:

a.	Two 2-noded isoparametric finite elements. 										(7)

b.	One 3-noded sub parametric element. 											(8)

Take E=100GPa and thickness =10mm

(OR)

19.	Determine the deflection and slope at 2m from the left end of a beam fixed at both the ends. A load of 1kN and a moment of 2kN-m acts on the beam as shown in the figure below. E=200GPa, I1=4mm4, I2=2mm4 .

20.	Determine the temperature (φ) at the point (P) located at (4,7) where (φi=400C), (φj=350C), and (φk=450C),  as shown in the figure below.

(OR)

21.	Derive the shape function for an eight noded rectangular element in (ξ,η)coordinate system.

22.	a.	Derive a shape function for a quadratic interpolation model for a one-dimensional element.  																					(8)

b.	Derive a shape function for a cubic interpolation model for a one-dimensional element.  																						(7)

(OR)

23.	Derive the shape function for nine noded rectangular element in (ξ,η)coordinate system.

[P.T.O]

24.	Explain briefly about following terms with its appropriate material constitutive relation matrix:

a.	Plane stress condition. 														(5)

b.	Plane strain condition. 														(5)

c.	Axis symmetric condition. 													(5)

(OR)

25.	Find the normal temperatures in a composite wall as shown in the figure below. The wall is maintained at 1000C at the left face and convection mode of heat transfer occurs between the right face and the existing fluid. K1=0.06 W/cm 0C    K2=0.2 W/cm 0C.