# Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

# Subject Title: ADVANCED MECHANISM DESIGN						  Time: 3 hours

# Subject Code: ME345					                 				 Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory	:

The four bar linkage of the figure shown below, has crank 2 driven by an input torque M12 an external load P=534 220N acts at point Q on link 4. For the particular position of the linkage shown, find all the constraint forces and their reactions necessary for this to be a position of equilibrium.

2. 	For the kinematic linkages shown in the figure below, calculate the following: the number of binary links; the number of ternary links; the number of other links; the number of total links; the number of loops; the number of joints or pairs; the number of degrees of freedom.

(OR)

[P.T.O]

3. 	Find the maximum and minimum transmission angles for the mechanisms shown in the figure below. The figures indicate the dimensions in standard units of length.

4. 	Perform a position analysis of the four bar linkage shown in the figure below by finding θ3 and θ4 for the two positions shown.

(OR)

5. 	Find the inflection circle for the motion of the coupler of the slider-crank linkage of the figure below and determine the instantaneous radius of curvature of the path of the coupler point C. All dimensions are in mm.

6. 	Explain the following in detail:

a.	Type Synthesis.																(4)

b.	Number Synthesis.															(4)

c.	Dimensional Synthesis.														(4)

d.	Function Generation.															(4)

e.	Path Generation.																(4)

(OR)

[P.T.O]

7.  	Synthesize a four bar linkage to give the following values for the angular velocities and accelerations.

ω2=200rad/s		ω3=85rad/s		ω4=130rad/s

α2=0rad/s2		α3=-1000rad/s2	α4=-1600rad/s2

8. 	A four link RGGR crank rocker mechanism is shown in the figure below. The known parameters are the position and plane of rotation of the input crank, the plane of rotation of the output crank, and the dimensions of all four links. Find the positions of all moving links when the input crank is set to θ2=-45° as shown.

(OR)

9. 	Briefly explain the Denavit – Hartenberg parameters with a neat sketch.