Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: ANALOG AND DIGITAL INTEGRATED CIRCUITS	      	       Time: 3 hours

Subject Code: MT226			       					               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	List any two features of cascode amplifier.

2.	Give any one disadvantage of CB amplifier.

3.	Why are differential amplifiers used in operational amplifiers?

4.	List the basic characteristic of an ideal differential amplifier.

5.	What is the significance of Bode plots?

6.	Name any two basic feedback topologies.

7.	Define Start of conversion.

8.	Give any two types of ADC.

9.	Mention the disadvantage of dynamic logic when compared to static logic.

10.	Give any two characteristic features of CMOS inverter.

PART – B  (5 x 3 = 15 MARKS)

11.	Compare the characteristics of MOSFET and BJT.

12.	Define CMRR.

13.	Give the pros and cons of negative feedback amplifiers.

14.	The basic step of a 9 bit DAC is 10.3 mV. If 000000000 represents 0V, what is the output

for an input of 101101111?

15.	List the characteristics of pass- transistor logic.

PART – C  (5 x 15 = 75 MARKS)

16.	Describe about the working of CE amplifiers with active load.

(OR)

17.	What is meant by emitter follower?  Explain its function in detail.

18.	Enumerate on the operation of BJT differential pair circuit.

(OR)

19.	Write notes on the function of multistage amplifiers.

20.	Explain the operation of shunt-series feedback amplifier with a circuit diagram.

(OR)

21.	Describe how stability analysis is carried out using Bode plots.

22.	Explain the following performance parameters of 741 Op-amp circuit: Gain, Frequency

response and Slew rate.

(OR)

23.	Explain about any four types of A/D converter circuits.

24.	Give details of the function of CMOS logic gate circuits.

(OR)

25.	Explain the working of dynamic logic circuits.