Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: FUNDAMENTALS OF ELECTRONIC COMMUNICATION      Time: 3 hours

Subject Code: MT233			       						              	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Noise arising from thermal causes is called _______ noise.

2.	Noise arising at low frequency is known as _______ noise.

3.	What is the name of the condition to sustain oscillation in an oscillator?

4.	Double conversion receiver allows the receiver to have good ______.

5.	Modulation index ranges from _______ to ________.

6.	_______ is a technique used in the transmission of speech signal to reduce channel noise.

7.	In angle modulation linearity can be improved by using _______.

8.	In the field of telecommunication most widely used modulation is ______.

9.	The ratio _______ is the key parameter in the design of digital communications systems.

10.	The function of a channel encoder is to __________.

PART – B  (5 x 3 = 15 MARKS)

11.	The noise figure of an amplifier is 7dB. Calculate the output signal to noise ratio when the input signal to noise ratio is 35dB.

12.	An LC oscillator has L=400 µH, C1 =100 pF, and C2 = 300 pF. Determine the frequency of oscillation.

13.	An amplitude signal consists of a symmetrical triangular wave having zero DC component and peak-to-peak voltage of 11 V. It is used to amplitude modulate a carrier of peak voltage 10 V. Calculate the modulation index and the ratio of the side lengths L1/L2 of the corresponding trapezoidal pattern.

14.	Write a short note on detectors.

15.	Write a note on error control coding.

PART – C  (5 x 15 = 75 MARKS)

16. 	Describe in detail about Noise factor.

(OR)

17.	Explain the working principle of Class C amplifier and find its efficiency.

18.	Discuss the working principle of LC Oscillator with an example.

(OR)

19.	Explain the working principle of a Super heterodyne receiver.

20.	Describe about Amplitude Modulation Transmitter with a neat block diagram.

(OR)

21.	Elaborate about SSB Reception with a neat block diagram.

22.	Explain the working principle of an Angle modulation detector.

(OR)

23.	Describe the working principle of Pulse Amplitude Modulation.

24.	Discuss the generation of a PCM signal and find the signal to quantization error ratio.

(OR)

25.	Explain the parity encoding technique and its error corrections methods.