Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – April/May-2013

Subject Title: SIGNAL TRANSMISSION AND RADIATION				       Time: 3 hours

Subject Code: MT238						       	      		       	       	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	Define characteristics impedance.

2.	What is VSWR?

3.	What is the maximum bandwidth of a twisted line?

4.	The radio frequency range is _____________.

5.	Which type of propagation is dominant at Microwave frequency range?

6.	Define Secant law.

7.	What is an antenna?

8.	Write any two types of antenna used in microwave frequency.

9.	Define Snell’s law.

10.	Write any two optical sources.

PART – B  (5 x 3 = 15 MARKS)

11.	How are transmission lines used as circuit elements?

12.	What are the different modes in wave guides?

13.	What is meant by isotropic source and isotropic medium?

14.	What is the relationship between effective area of aperture and power gain of an antenna?

15.	What are the advantages and disadvantages of fiber optic communication system?

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail about slotted line measurements at radio frequencies.

(OR)

17.	Using a Smith chart, calculate the position and length of a stub designed to match a 100 Ω load to a 50 Ω line, the stub being short circuited. If this matching is correct at 63 MHz, what will be the SWR on the main line at 70 MHz? Note that the load is a pure resistance.

18.	Explain in detail about rectangular wave guides and circular wave guides.

(OR)

19.	Write a short note on the following:

a.	Microstrip lines						b.	Telephone lines

20.	Describe briefly the strata of the ionosphere and their effects on sky wave propagation. Why is this propagation generally better at night than during the day?

(OR)

21.	Explain in detail about surface wave propagation.

22.	Discuss in detail about dipole antenna and derive its far field components.

(OR)

23.	Explain in detail about radio frequency antennas.

24.	Draw the block diagram of the optical communication system and explain in detail.

(OR)

25.	With neat sketches explain the operation of a photo detector in detail