Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 13EC101 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BASIC ELECTRONICS ENGINEERING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Discuss the formation of N type and P type semiconductor with its covalent bond structure. | CO1 | 15 |
|  | b. | Elaborate on the active and passive components with an example. | CO1 | 5 |
| (OR) |  |  |  |  |
| 2. | a. | Describe the biasing and characteristics of rectifier diode with a neat diagram. | CO1 | 12 |
|  | b. | Explain Zener diode voltage regulator circuits. | CO1 | 8 |
| 3. |  | Elucidate on the various logic gates with its truth table and expression. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Design and explain the working of half adder with its truth table. | CO1 | 10 |
|  | b. | Simplify the following using a K-map F(A,B,C,D)=Σ(0,1,2,3,5,7,9,11,12,13,14,15). | CO1 | 10 |
| 5. | a. | Paraphase the operation of common emitter circuit and sketch the input and output characteristics. Also explain operating regions by indicating them on characteristics curve. | CO1 | 15 |
|  | b. | State the difference between BJT and FET. | CO1 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | Discuss in detail the construction of N-channel JFET and explain briefly its drain characteristics. | CO1 | 12 |
|  | b. | With a neat circuit and necessary equations, explain the voltage follower. | CO1 | 8 |
| 7. | a. | List out the types of analog modulation with an example. | CO2 | 5 |
|  | b. | With a neat block diagram, explain the working of super heterodyne receiver. | CO2 | 15 |
| (OR) |  |  |  |  |
| 8. | a. | Discuss in detail about amplitude modulation. Also derive the expression of the amplitude modulated wave. | CO2 | 15 |
|  | b. | Comprehend in detail about the need for modulation. | CO2 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | Discuss briefly about the block of uplink and downlink conversion in satellite communication with a neat sketch. | CO2 | 10 |
|  | b. | Elaborately discuss about the fibre optic communication. | CO2 | 10 |