Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14AE2014 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # AIRCRAFT PERFORMANCE | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Discuss the flow over a bluff body with relevant graph. | CO1 | 10 |
|  | b. | Define induced drag. For a finite wing show that CD = Cd + CL2/(πeAR), where Cd = profile drag coefficient, e = span efficiency factor and AR = aspect ratio. | CO3 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Consider the flow over a small plate that is 5cm long in the flow direction and 1cm wide. The free stream conditions corresponds to standard sea level.The flow velocity is 120 m/s and the viscosity µ = 1.789 × 10-5 kg/m.s . Assuming laminar flow, calculate - The boundary layer thickness at the downstream edge ( the trailing edge) - The drag force on the plate | CO1 | 10 |
|  | b. | Discuss the pressure distribution over an airfoil at different angle of attack with relevant graph. | CO1 | 10 |
| 3. | a. | Describe the concept of power available and power required for propeller driven aircraft. | CO3 | 10 |
|  | b. | A single engine propeller driven airplane has following characteristics: span =10.91 m, wing area = 16.16 m2 , W = 1340 kg, e = 0.8, CDo = 0.022. Calculate the thrust required at 12000 m for flight velocity of 120 m/s. | CO3 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Obtain an expression for rate of climb for steady unaccelerated climbing flight. | CO3 | 10 |
|  | b. | Obtain an expression for equilibrium glide velocity V∞. | CO3 | 10 |
| 5. | a. | For a jet propelled airplane, derive the Brequet range equation. List the conditions for obtaining maximum range. | CO3 | 15 |
|  | b. | Define service and absolute ceilings. | CO1 | 5 |
| (OR) |  |  |  |  |
| 6. |  | A jet powered aircraft has the following characteristics, wing span = 15m, wing area 25m2 , normal gross weight = 88176.75 N, fuel capacity = 1119 gal of kerosene,TSFC = 1.667 × 10-4 s-1, CDo = 0.02 and e = 0.81. Assuming the cruising altitude is 6705.6 m Calculate the endurance and range of the aircraft. | CO3 | 20 |
| 7. | a. | Describe the various phases of take-off flight and derive the expression to estimate the take-off ground roll distance of an aeroplane. | CO3 | 10 |
|  | b. | With the help of neat sketch, explain the V-n diagram. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Derive the expression to estimate the landing ground roll distance. | CO3 | 10 |
|  | b. | A sailor weighs 45,000N and has a wing loading of 600 N/m2. The drag equation CD= 0.700+0.022CL2, estimate the gliding angle and rate of descent, when driving with air brake, extended at 30 m/sec at sea level. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Derive the expression for the following maneuvers. i) Pull up ii) Pull down | CO2 | 10 |
|  | b. | The maximum lift to drag ratio for the CP-1 is 13.6, W = 2950 kg, s = 174 m2. Calculate the minimum glide angle and maximum range measured along the ground, covered by CP-1 in a power off glide that starts at altitude of 3048 m. | CO2 | 10 |