Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14AE2014 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # AIRCRAFT PERFORMANCE | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Explain with neat sketch the pressure distribution around the airfoil at different angle of attack. | CO1 | 10 |
|  | b. | Write short notes on: Skin friction drag, pressure drag, and induced drag. | CO3 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Consider the flow over a small plate that is 5 cm long in the flow direction and 1 cm wide. The free stream conditions corresponds to standard sea level. The flow velocity is 120 m/s and the viscosity µ = 1.789 × 10-5 kg/m.s . Assuming laminar flow, calculate - The boundary layer thickness at the downstream edge ( the trailing edge) - The drag force on the plate | CO1 | 10 |
|  | b. | Define the term aerodynamic center and centre of pressure and derive an expression to locate the aerodynamic center. | CO1 | 10 |
| 3. | a. | Describe the concept of power available and power required for propeller driven aircraft. | CO3 | 10 |
|  | b. | By analytical approach show that the minimum power required occurs when the airplane is flying such that CL3/2/CD and plot the variation of PR Vs V∞. | CO3 | 10 |
| (OR) |  |  |  |  |
| 4. |  | An aircraft weighing 250000N has a wing area of 80 m2 and its drag equation is CD= 0.016 +0.04 CL2. Calculate the minimum thrust and the minimum power required for straight and level flight, at 11500 m. The standard sea level values are pressure = 101325 N/ m2, density = 1.2256 Kg/m3 and temperature = 288.16 K, lapse rate = -0.0065 K/m. | CO3 | 20 |
| 5. | a. | For a propeller driven airplane derive the Brequet range equation. List the conditions for obtaining maximum range. | CO3 | 10 |
|  | b. | Obtain the expression for rate of climb for steady, unaccelerated climbing flight. | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. |  | A jet powered aircraft has the following characteristics, wing span = 15 m, wing area 25 m2 , normal gross weight = 88176.75 N, fuel capacity = 1119 gal of kerosene,TSFC = 1.667 × 10-4 s-1, CDo = 0.02 and e = 0.81. Assuming the cruising altitude is 6705.6 m Calculate the endurance and range of the aircraft. | CO3 | 20 |
| 7. | a. | Describe the various phases of take-off flight and derive the expression to estimate the take-off ground roll distance of an aeroplane . | CO3 | 10 |
|  | b. | With the help of neat sketch, explain the V-n diagram. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Derive the expression to estimate the landing ground roll distance. | CO3 | 10 |
|  | b. | Estimate ground roll of an aeroplane at SL with no thrust reversal. However spoilers are used such that L = 0. The spoilers increase the zero lift drag co-efficient by 10% .The fuel tanks are empty. The maximum lift co-efficient 2.5, given W = 5601.1 kg, S = 29.55 m2, CDo = 0.02, µ = 0.4 | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. | a. | Derive the expression for the following maneuvers. i) Pull up ii) Pull down | CO2 | 10 |
|  | b. | The maximum lift to drag ratio for the CP-1 is 13.6, W = 2950 kg, s = 174 m2. Calculate the minimum glide angle and maximum range measured along the ground, covered by CP-1 in a power off glide that starts at altitude of 3048 m. | CO2 | 10 |