Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14AE2015 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # AIRCRAFT STABILITY AND CONTROL | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Derive the contribution of aircraft horizontal tail in stick fixed static longitudinal Stability. | CO2 | 15 |
|  | b. | How is stick-fixed neutral point determined from flight tests? |  | 5 |
| (OR) |  |  |  |  |
| 2. |  | The hinge moment characteristics of the horizontal tail of an airplane are : Chαt = - 0.003 deg-1, Chδe = - 0.0055 deg-1 ,Chαt = - 0.003 deg-1. Obtain the floating angle of elevator (δefree ) for αt = 5 and 10. Assume δt = 4 | CO2 | 20 |
| 3. | a. | Define and derive the term Elevator Effectiveness. | CO2 | 6 |
|  | b. | Derive the expression for Elevator trim angle. |  | 8 |
|  | c. | Define and derive an expression for stick force gradient (dF/dV) and explain the requirement for proper stick force variation. |  | 6 |
| (OR) |  |  |  |  |
| 4. |  | For an airplane the pitching moment co-efficient is given by the following expression when c.g. lies at 0.25 c . Cmcg = 0.05 - 0.10CL - 0.01δe where δe is in degrees. Answer the following: - Is the airplane statically stable? Justify. - What is the static margin in this case? - What is the location of neutral point stick fixed? - What is the value of CL for which equilibrium is achieved with zero elevator deflection? - What is the value of elevator effectiveness (Cmδ)? - If the elevator deflection is limited to ± 250, locate the most forward c.g. location for which trim is obtained at CL=1.5 | CO2 | 20 |
| 5. | a. | Derive the contribution of sweep back wing towards the static directional stability. | CO2 | 10 |
|  | b. | How does the Adverse Yaw occurs and Elaborate how to avoid it. |  | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Derive the required Rudder angle for Equilibrium condition. | CO3 | 10 |
|  | b. | Explain the following: - One Engine inoperative condition. - Rudder Lock. - Weather cock Stability. |  | 10 |
| 7. | a. | Define and derive the expression for Aileron power. |  | 6 |
|  | b. | Define Dihedral Angle and Explain the effects in lateral Stability. |  | 4 |
|  |  | Explain the phenomena Aileron Reversal. |  | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Derive the Equation of motion for dynamic longitudinal stability. | CO3 | 14 |
|  | b. | Explain the term Routh’s Criteria. |  | 6 |
|  |  | Compulsory: |  |  |
| 9. |  | Explain the following term - Dutch Roll. - Auto Rotation. | CO3 | 20 |