Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14AE2016 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SPACE DYNAMICS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Write Kepler’s laws of motion. Use third law of motion to calculate the orbital period of Mars if its distance from Sun is 1.52 Astronomical Units (AU). | CO2 | 5 |
|  | b. | Find the orbital period of an Earth satellite in minutes whose semi-major axis (a) is 8500 km. Gravitational constant (µ) for Earth =398600 km3/s2. | CO2 | 3 |
|  | c. | From the Kepler’s equation M = E – e sin E, where e is the eccentricity of an elliptic orbit and E and M are eccentric and mean anomaly, respectively, if M = 80 degrees and e = 0.4, calculate the eccentric anomaly E in degrees. | CO2 | 8 |
|  | d. | # Compute the eccentric anomaly E from the true anomaly θ and the eccentricity e using the following relations # cos E = ( e + cos q) / (1 + e cos q), # sin E = (1-e2)1/2 sin q / (1 + e cos q), for e = 0.2 and q = 100 degrees. | CO2 | 4 |
| (OR) |  |  |  |  |
| 2. | a. | Draw a neat diagram to show the six orbital elements of a satellite moving in an elliptic orbit. | CO2 | 4 |
|  | b. | If the position and velocity of a satellite are (-6040, -3500, 2505) km and (-3.46, 6.62, 2.52) km/s, respectively; find the angular momentum and the orbital elements: eccentricity (e), inclination (i), argument of perigee (ω), right ascension of ascending node (Ω) and true anomaly of the satellite. | CO2 | 16 |
| 3. | a. | Explain briefly geocentric-inertial coordinate system. | CO2 | 3 |
|  | b. | Define Sun-synchronous orbit. Calculate the orbital inclination for an elliptic Sun-synchronous orbit, whose semi-major axis is 7800 km and eccentricity is 0.1.Earth’s gravitational constant (μ) = 398600 km3s-2, J2= 0.00108263 and Earth’s radius is 6378 km. | CO2 | 10 |
|  | c. | If the mean anomaly and eccentricity for the Kepler’s equation for hyperbola Mh = e sinh F – F, are 105 radians and 1.5, respectively, calculate the eccentric anomaly F. Approximate value for F is 5 radians. | CO2 | 7 |
| (OR) |  |  |  |  |
| 4. | a. | Explain Cowell’s and Encke's methods. Give their advantages and disadvantages. | CO2 | 14 |
|  | b. | Write four important perturbing forces acting on an Earth satellite. Explain briefly any two of them. | CO2 | 6 |
| 5. | a. | Find the additional velocity required for a Hohmann transfer from circular Earth satellite orbit of radius 7200 km to a circular Earth satellite orbit of radius 8200 km. | CO2 | 9 |
|  | b. | Calculate the velocity change required to transfer a satellite from a circular orbit of 500 km altitude with an inclination of 40 to an orbit of the same size at an inclination of 20. Earth’s gravitational constant = 398600 km3s-2. | CO2 | 5 |
|  | c. | Calculate the synodic period of Venus relative to the Earth. The orbital periods of Earth and Venus are 365.26 days and 224.7 days, respectively. | CO2 | 3 |
|  | d. | Calculate the radius of sphere of influence of the Mars. The mass of the Mars and the Sun are 6.419x1023 kg and 1.989 x1030 kg, respectively. The radius of Mars’s orbit about Sun is 227.9x106 km. | CO2 | 3 |
| (OR) |  |  |  |  |
| 6. | a. | Prove that the radial velocity vr = µ e sin θ / h. | CO2 | 3 |
|  | b. | The perigee radius of a satellite in a parabolic geocentric trajectory is 7000 km. Find the distance between the points P1 and P2 on the orbit which are 9000 and 15000 km, respectively, from the centre of the Earth. Earth’s gravitational constant = 398600 km3s-2. | CO2 | 5 |
|  | c. | At a given point of a spacecraft’s geocentric trajectory, the radius (r) is 14000 km, the speed (v) is 8.7 km/s, and the flight path angle (γ) is 50 degrees. Show that the path is a hyperbola. Calculate the hyperbolic excess velocity, angular momentum, true anomaly and eccentricity. Earth’s gravitational constant = 398600 km3s-2. | CO2 | 9 |
|  | d. | For a spacecraft trajectory around the Earth, the radial distance r = 14000 km when the true anomaly θ = 40 degrees, and r = 31000 km when θ = 95 degrees. Calculate the eccentricity e of the orbit. | CO2 | 3 |
| 7. | a. | Describe briefly Earth's atmosphere. Show in a figure the change in temperature up to 120 km altitude. | CO2 | 10 |
|  | b. | # A geocentric trajectory has perigee velocity of 14 km and perigee altitude of 222 km. Find its eccentricity. Find the radius vector when the true anomaly is 75 degrees. Earth’s gravitational constant is 398600 km3s-2. | CO2 | 6 |
|  | c. | # Estimate the trip time T from the Earth to Mars in days along the Hohmann transfer orbit by assuming the orbits of Earth and Mars around the Sun to be circular with radii of 149.6 x 106 and 227.9 x 106 km, respectively. The value of the Sun’s gravitational constant (µ) = 1.32715 x 1011 km3s-2. | CO2 | 4 |
| (OR) |  |  |  |  |
| 8. | a. | Explain static stability margin. | CO1 | 4 |
|  | b. | Explain the purpose of fins on a rocket. | CO1 | 4 |
|  | c. | Explain static and dynamics stability of rockets. | CO1 | 12 |
|  |  | Compulsory: |  |  |
| 9. | a. | From the first principles, derive the rocket equation Vb = g0Ispln(Mi/Mf), Where Vb is the burnout velocity, g0 is the acceleration due to gravity at sea level, Isp is specific impulse and Mi/Mf is the mass ratio. | CO1 | 7 |
|  | b. | If the Isp of a rocket using hydrogen and oxygen as fuel and oxidizer is 430 s, and Vb is 10000 metres/s, calculate its mass ratio (g0= 9.8 m/s2). | CO1 | 4 |
|  | c. | A two-stage rocket has the following design characteristics. First stage: propellant mass = 30000 kg, structural mass = 12000 kg. Second stage: propellant mass = 15000 kg, structural mass = 4500 kg. The payload mass is 150 kg. The specific impulse for first stage is 290 s and for the second stage is 370 s. Calculate the final burnout velocity(g0=9.8 m/s2). | CO1 | 9 |