Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14AE2017 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # AIRCRAFT PROPULSION | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | # Draw the P-V and T-S diagram for Turbojet engine. | CO1 | 6 |
|  | b. | # In a gas turbine plant air enters the compressor at 1 bar and 7˚C. It is compressed to 4 bar with an isentropic efficiency of 82%. The maximum temperature at the inlet to the turbine is 800˚C. The isentropic efficiency of the turbine is 85%. The calorific value of the fuel used is 43.1 MJ/kg. The heat losses are 15% of the calorific value. Calculated the following: (i) Thermal efficiency (ii) Air/Fuel ratio (iii) Specific fuel consumption in kg/kW/hr. Assume Cpa=1.005 kJ/kg, γa=1.4, Cpg=1.147 kJ/kg, γa=1.4. | CO2 | 14 |
| (OR) |  |  |  |  |
| 2. | a. | # State the important aspects and applications of Brayton Cycle. | CO2 | 4 |
|  | b. | # A turboprop aircraft is flying at 600 km/hr at an altitude where the ambient conditions are 0.458 bar and -15˚C. Compressor pressure ratio is 9 and maximum gas temperature 1200K. The intake duct efficiency is 0.9 and total head isentropic efficiency of compressor and turbine is 0.89 and 0.93 respectively. Calculate the specific power output in kJ/kg, thermal efficiency of the unit taking mechanical efficiency of transmission as 98% and neglecting the losses other than specified. Assume that exhaust gases leave the aircraft at 600km/hr relative to the aircraft. | CO2 | 16 |
| 3. | a. | # Classify the Thrust Augmentation methods used in aircraft. | CO2 | 6 |
|  | b. | Define cycle and explain how practical cycle is different from actual cycle of Simple Gas turbine engine? | CO2 | 4 |
|  | c. | # Explain in detail about Ramjet engine and its working principle with neat sketch. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | # A centrifugal compressor has an inlet eye 15 cm diameter. The impeller revolves at 20,000rpm and the inlet air has an axial velocity of 107 m/s, inlet stagnation condition temperature 294K and inlet pressure 1.03kg/cm2. Determine # Theoretical blade angle at this point and # Mach number of the flow at the tip of the eye. | CO2 | 8 |
|  | b. | # With neat sketch explain the essential parts and working principle of centrifugal compressor. | CO1 | 12 |
| 5. | a. | # With neat sketch explain the combustion chamber geometry bringing out the various zones that play a part in the process of combustion. | CO1 | 12 |
|  | b. | # Write short notes on # Flame tube Cooling, # Fuel injection # Ignition # Use of cheaper fuels and # Pollution. | CO1 | 8 |
| (OR) |  |  |  |  |
| 6. |  | An axial flow air compressor of 50% reaction design has blades with inlet and outlet angles of 45°and 10° respectively. The compressor is to produce a pressure ratio of 6:1 with an overall isentropic efficiency of 0.85 when inlet static temperature is 37°C. The blade speed and axial velocity are constant through the compressor. Assuming a value of 200m/s for blade speed find the number of stages required if the work done factor is i) unity and ii) 0.87 for all stages. | CO1 | 20 |
| 7. | a. | Explain the principle of operation of a turbine and what are the primary parts of a turbine? | CO1 | 10 |
|  | b. | With a neat sketch explain a single stage velocity triangle and derive an expression for the work output. | CO2 | 10 |
| (OR) |  |  |  |  |
| 8. |  | In a single-stage turbine the nozzle discharges the fluid on to the blades at an angle of 65° to the axial direction and the fluid leaves the blades with an absolute velocity of 300m/s at an angle of 30° to the axial direction. If the blades have equal inlet and outlet angles and there is no axial thrust, estimate the blade angle, power produced per kg/s of the fluid and the blades efficiency. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Summarize the functions of an exhaust nozzle. | CO1 | 10 |
|  | b. | Write short notes on - engine back pressure control, - exhaust nozzle area ratio, and - thrust reversing. | CO1 | 10 |