Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 14AE2026 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | WIND TUNNEL TECHNIQUES | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Give a detail description about the parts and working principle of closed circuit subsonic wind tunnel. | CO1 | 20 |
| (OR) |  |  |  |  |
| 2. | a. | Explain about the significance of non dimensional numbers. | CO1 | 10 |
|  | b. | Consider flow over two circular cylinders, one having four times the diameter of the other. The flow over the smaller cylinder has a freestream density, velocity and temperature given by ρ1, V1 and T1 respectively. The flow over the larger cylinder has a free stream, velocity and temperature given by ρ2, V2 and T2 respectively where ρ2 = ρ1/4, V2 = 2V1, T2 = 4T1. Assume both μ and a are proportional to T0.5. Show that two flows are dynamically similar. | CO1 | 10 |
| 3. |  | Explain about intermittent indraft wind tunnels with a neat sketch. | CO1 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Write short notes on calculating the run time for intermittent blow down type tunnels. | CO2 | 10 |
|  | b. | A supersonic wind tunnel has storage tank and stagnation pressure as 20 bar and 4 bar respectively. Estimate the runtime available if the tunnel is running at Mach 2 and stagnation temperature of 300 k. Take γ = 1.4, volume of the storage tank 14.14 m3 exit area of the nozzle be 100 X 100 mm2. | CO2 | 10 |
| 5. |  | Explain about hypersonic wind tunnels with a neat sketch. | CO1 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Explain in detail about the shadowgraph flow visualization technique used in high speed wind tunnel. | CO3 | 20 |
| 7. |  | Explain the working of shock tube with neat sketch. | CO2 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Explain in detail about laser doppler velocimetry with a neat sketch. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Explain the concept of a six component balance with a neat sketch. | CO2 | 20 |