Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14BT2003 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # PRINCIPLES OF CHEMICAL ENGINEERING | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Unit conversion: 1m3 = …..………….. lt , 1 bar = ……………. pa 1 ft2 = ….…………………… in2 1 torr = …………..………… mm Hg | CO1 | 10 |
|  | b. | What is the average molecular weight of air and its composition by weight? | CO1 | 6 |
|  | c. | In a double effect evaporator plant, the second effect is maintained under vacuum of 500 torr. Find the absolute pressure in kpa, bar and psi | CO1 | 4 |
| (OR) |  |  |  |  |
| 2. |  | Cracked gas from a petroleum refinery has the following composition by volume methane 50%,ethane 15%,ethylene 5%,propane 10%,propylene 10% and n-butane 10%. Find (a).the average molecular weight of the gas mixture (b).the composition by weight (c).specific gravity of the gas mixture | CO 1 | 20 |
| 3. |  | # A multiple effect evaporator system has a capacity of processing10,000 kg/day of solid caustic soda. When it concentrates weak liquor from 4 to 25% by weight caustic soda. When the same plant is fed with 10% weak liquor and if it is concentrated to 50% (both in weight basis).find the capacity of the plant in terms of solid caustic soda? Assume that the water evaporating capacity to be same in both cases. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. | a. | Dilute ethyl alcohol is concentrated using a distillation column. The feed is 1,000 kg/hr of dilute alcohol with 25% alcohol by weight. The bottom product contains 0.4% alcohol by weight. The product contains 94% alcohol by weight. Find the percentage loss of water to the total water in the feed | CO1 | 10 |
|  | b. | A bottle contains 5 kg water and 5 kg sodium hydroxide. What are the mass fraction and mole fraction of each component in the bottle? | CO1 | 10 |
| 5. | a. | Dryer system handles 500 kg/day of wet solids. Wet solids containing 80% solids and the product coming out has 2% moisture. Calculate the percentage of original water that is removed in the product. | CO3 | 10 |
|  | b. | Write the formulae of Heat of content in terms of formation | CO3 | 5 |
|  | c. | Write the formulae of Heat of reaction | CO3 | 5 |
| (OR) |  |  |  |  |
| 6. | a. | When liquid benzene is completely burned to carbon dioxide and liquid water, the standard heat of combustion is – 3267.6 kJ/mol. The standard heat of combustion of hydrogen to liquid water is – 285.83 kJ/mol and that of carbon to carbon dioxide gas is – 393.51 kJ/mol. Calculate the standard heat of formation of liquid benzene. | CO2 | 15 |
|  | b. | # Calculate the heat that must be removed in cooling 32 kg of oxygen from 488K to 313K using Cp° data: # Cp° = a+bT+cT2+dT3 , (kJ/kmol.K) # Gas; # a; # b x 103; # c x 106; # d x 109. # Oxygen; # 26.0257; # 11.7551; # - 2.3426; # - 0.5623. | CO2 | 5 |
| 7. |  | # The gases leaving the boiler at 523K have the following composition: CO2 = 11.31%, H2O=2.17%,O2= 13.04% and N2=73.48% by volume. Calculate the heat lost in 1 kmol of gas mixture above 298K,using the heat capacity data given below: Cp° = a+bT+cT2+dT3 , (kJ/kmol.K) # Gas; # a; # b x 103; # c x 106; # d x 109. # SO2; # 21.3655; # 64.2841; # - 41.0506; # 9.7999. # SO3; # 32.4921; # 0.0796; # 13.2107; # - 4.5474. # O2; # 26.0257; # 11.7551; # - 2.3426; # - 0.5623. # N2; # 29.5909; # - 5.141; # 13.1829; # - 4.968. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | # Flue gases leaving the boiler stack at 523K have the following composition: CO2 = 111.31%,H2O = 13.04%,O2 = 2.17% and N2 = 73.48% (by volume). Calculate the heat lost in 1 kmol of gas mixture above 298K using the heat capacity data given below: Cp° = a+bT+cT2+dT3 , (kJ/kmol.K) # Gas; # a; # b x 103; # c x 106; # d x 109. # CO2; # 21.3655; # 64.2841; # - 41.0506; # 9.7999. # H2O; # 32.4921; # 0.0796; # 13.2107; # - 4.5474. # O2; # 26.0257; # 11.7551; # - 2.3426; # - 0.5623. # N2; # 29.5909; # - 5.141; # 13.1829; # - 4.968. | CO3 | 15 |
|  | b. | # Calculate the heat of formation of gaseous n-heptane at 298.15 K using the following data: # Data: # Standard heat of formation of CO2= - 393.51 kJ/mol # Standard heat of formation of H2O = - 285.83 kJ/mol # Standard heat of combustion of C7H16 = - 4853.43 kJ/mol | CO3 | 5 |
|  |  | Compulsory: |  |  |
| 9. |  | # Calculate the heat of formation of gaseous ethyl alcohol at 298.15K using the following data: Standard heat of formation of carbon dioxide = -393.51 kJ/mol. Standard heat of formation of water = -285.83 kJ/mol Heat of combustion of gaseous ethyl alcohol at 298.15K = -1410.09kJ/mol | CO3 | 20 |