Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14BT2009 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # BIOPROCESS PRNCIPLES | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. |  | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | Summarise five groups of commercially important fermentation process available. Relate in detail with examples. | CO1 | 20 |
| (OR) |  |  |  |  |
| 2. |  | Elaborate various stages in development of fermentation industry also tabulate various process controls, vessels used and mode of operation of these process in detail. | CO1 | 20 |
| 3. |  | Explain in detail the process of medium formulation and the important constituents need to be added for formulating an industrial scale medium for Penicillin formulation. | CO3 | 20 |
| (OR) |  |  |  |  |
| 4. |  | Illustrate with an example the Plackett Burmann method of medium optimization with Nelson’s example. | CO3 | 20 |
| 5. |  | Design the sterilization holding time for an industrial scale batch medium sterilization process with an example. | CO2 | 20 |
| (OR) |  |  |  |  |
| 6. |  | Air is sterilized through a depth filter and is sent at an flow rate of 17 m3/min for an fermentation process for 460x105min with an linear velocity of 0.17m/min. The value of the rate constant is 1.54 m-1 Calculate # Initial number of microorganism present in air. # Radius of the filter. # Length of the filter. # Cross sectional area of filter. | CO2 | 20 |
| 7. |  | Explain in detail the preservation techniques followed to store isolated industrially important microbes. | CO3 | 20 |
| (OR) |  |  |  |  |
| 8. |  | Describe with a neat diagram the process of inoculum development for brewing process. | CO3 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | Illustrate with a neat sketch the working of various types of continuous sterilization equipments in detail. | CO2 | 20 |