Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 14BT2022 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | UNIT OPERATIONS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | A crusher employed in mineral ore unit accepts feed having a volume-surface mean diameter of 15mm and gives a product of volume surface mean diameter of 5mm.The power required to crush 16tonnes/hr of the feed material in 9hp. Determine the power consumption if capacity is reduced to 10tonnes/hr. | CO1 | 10 |
|  | b. | Write on 3 different crushers with diagram. | CO2 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | A brick material is crushed in an Blake jaw crushers. The average size of the particles is reduced from 5cm to 1.3cm with a consumption of 36 Wh/metric tonne. What will be the consumption of energy necessary to crush the same material of average size of 7cm to an average size of 3cm? You may assume that the mechanical efficiency remains unchanged. Do the workout using - Rittingers Law. - Kicks Law. | CO1 | 12 |
|  | b. | Derive the Angle of nip for crushers roll. | CO2 | 8 |
| 3. | a. | 1 tonne per hour dolomite is produced by a ball mill operating in a closed circuit grinding with 100 number screen. The screen analysis (wt%) is given below. Calculate percentage screen efficiency. Mesh number; Feed%; Oversize%; Undersize%. 35; 7.07; 13.67; 0.00. 48; 16.60; 32.09; 0.00. 65; 14.02; 27.12; 0.00. 100; 11.82; 20.70; 2.32. 150; 9.07; 4.35; 14.32. 200; 7.62; 2.07; 13.34. -200; 33.80; 0.00; 70.02. 100; 100; 100. | CO1 | 10 |
|  | b. | Derive screen effectiveness and describe what is importance of mesh number. | CO2 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | For a sludge filter is a plate and frame filter mess, the filtration equation is V2=kt, where V is volume & t is time where the pressure is constant 30m3 of filtrate is obtained in 10hrs.Calculate the washing time if 3m3 of wash water is forced to the cake at the end of filtration. | CO1 | 10 |
|  | b. | Derive the constant pressure & constant volume filtration process. | CO2 | 10 |
| 5. | a. | Calculate the specific cake resistance and the resistance of the filter medium on the basis of the following experimental data, for constant pressure filteration.Filteration of a suspension of incompressible solids of concentration 30Kg/m3 on a filter medium area of 1m2. The pressure drop was 2 bars and the viscosity of filterate is 2.1x10-3 Kg/m.s Volume of filterate (m3); Time (t). 2 x10-3; 24. 4 x10-3; 59. 6 x10-3; 111. 8 x10-3; 184. 10 x10-3; 270. | CO1 | 12 |
|  | b. | Elaborate on filteration & filteration types | CO2 | 8 |
| (OR) |  |  |  |  |
| 6. | a. | A soil paste containing 13% moisture was mixed in a large Muller mixer with 10 weight percent of a tracer consisting of dextrose and picric acid, after 3 mins f mixing 13 random samples was taken from the mix and analysed colorimetrically for tracer material.The measured cake in the sample was in weight % 10.33, 9.28, 7.84,10.24, 11.08, 10.02, 11.92, 9.72, 9.19, 10.75, 10.55. Calculate mixing index. | CO1 | 12 |
|  | b. | Elaborate mixing and type of impellers. | CO2 | 8 |
| 7. | a. | Calculate the maximum velocity at which spherical particle of silica 0.04mm diameter will fall through water at temperature 740C. Specific gravity of silica is 2.7 and specific gravity of water is 1. Viscosity of water is 0.3cpoise | CO3 | 10 |
|  | b. | Write in detail on gravity settling and gravity classifiers. | CO4 | 10 |
| (OR) |  |  |  |  |
| 8. |  | What is agitation? Discuss in detail about agitation equipment with a diagram. | CO4 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Calculate the maximum velocity at which spherical particle of Galena 0.15cm diameter will fall in water. where Density of water is 1kg/m3. Density of Galena is 7.5 viscosity of water is 0.3cpoise Drag coefficient is 0.45 | CO3 | 10 |
|  | b. | Discuss and derive the Dimensional analysis of Agitation. | CO4 | 10 |