Reg.No. ____________

End Semester Examination – April /May – 2018

| # Code : | # 14BT3009 | # Duration: | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # ADVANCED PROCESS EQUIPMENT DESIGN | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

(Any missing data may be suitable be assumed, Extra graph sheet allowed)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | Draw ten unit operations symbols with mass balances. | CO1 | 10 |
|  | b | Discuss about the location of plant layout. | CO1 | 10 |
| (OR) |  |  |  |  |
| 2. | a. | Explain in detail the design of Storage vessel for several hundreds of atmospheric pressure. (Horizontal vessel). | CO1 | 10 |
|  | b | Describe the engineering materials properties and give its industrial applications. | CO1 | 10 |
| 3. | a. | Explain the designing steps for Venturimeter and give its industrial application. | CO2 | 10 |
|  | b. | A venturimeter is used to measure the flow rate of water flowing in a pipe line of 78mm i.d . The throat diameter is 15mm, mercury manometer reads 18cm. Calculate the flow rate of water if Cv=0.96, Density of water and mercury is 1000 and 13600 kg/m3. | CO3 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | Write short notes on Different valves (minimum 3). | CO2 | 10 |
|  | b. | Write advantage and disadvantages of venturimeter and orifice meter. | CO1 | 10 |
| 5. | a. | Cold fluid is flowing through the heat exchanger at a rate of 15 m3/hr. It enters the heat exchanger at 303K and leaves at 328K.The hot thermic fluid enters the heat exchanger at the rate of 21m3/hr at a temperature of 388K .Find out the area of heat transfer required assuming the flow is counter current and overall heat transfer coefficient be 2490 W/m2 k. Data : Density of cold fluid= 1000 kg /m3, density of thermic fluid = 950 kg/m3,Specific heat of cold fluid= 4.187 kJ/kg.K, Specific heat of thermic fluid=2.93 kJ/kg.K. Draw a neat block diagram | CO3 | 10 |
|  | b. | Describe the design procedure about single effect evaporator with neat sketch. | CO3 | 10 |
| (OR) |  |  |  |  |
| 6. | a. | Write down the design procedure of shell and tube heat exchanger with neat sketch. | CO2 | 10 |
|  | b. | Calculate the amount of steam required for concentrating the solution of caustic soda from 10% by weight of solids to 40% by weight of solids in a single effect evaporator. The feed rate in 25000 kg/hr and its temperature is 60ºC. The absolute pressure in the evaporator is 0.2 kg/ cm2. Saturated steam at 1.4 kg/ cm2 is to be used as heating medium. The elevation in boiling points is 25 ºC. If the overall heat transfer coefficient is 670 kcal/ hr m2 ºC. Calculate the heating surface required and steam economy. The enthalpy data for various streams are as follows. Vapour at 0.2 kg/ cm2 = 623 kcal/ kg. 28% NaOH at 60 ºC = 50 kcal / kg, 40% NaOH at 85 ºC =90 kcal / kg. Latent heat of steam at 1.4 kg/ cm2 =534 kcal / kg. | CO3 | 10 |
| 7. | a. | A mixture of 65mole % A and 35Mole % B is to be separated in a fractionating column. The concentration of A in the distillate is 97Mole % and 65% of all product A is in the distillate. The feed is half vapor and the reflux ratio is to be 2.0.The relative volatility of A to B is 2.4. Calculate the number of theoretical plates in the column and locate the feed plate. If efficiency is 65%, justified the Actual plates required. | CO3 | 10 |
|  | b. | 1000 kg moles/hr of an ethanol propanol mixture containing 65moles% is separated in a continuous plate column operating at 1 atmosphere total pressure. The desired terminal composition in units of mole fraction of ethanol are XD=0.92, Xw=0.07.The feed is a saturated vapour and total condenser is used. When the reflux flow rate is four times the amount of top product, find the number of theoretical plates required for the separation. Relative volatility=2.10. | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | Describe the working of rotary drum dryer with neat diagram and give its industrial Applications. | CO2 | 10 |
|  | b. | A bubble cap fractionating column of 12 plates working at an average efficiency of 75% is being used to distill. 1000kg/hr of aqueous methanol at its bubble point enters the tower. The feed, over head product & bottom product are 50 mole %, 90 mole% and 10 mole% methanol. A total condenser is provided. The reflux is sent at its saturation temperature. If the reflux ratio is 1.7 times the minimum check whether the column available is satisfactory. The relative volatility of A to B is 2.4. | CO3 | 10 |
|  |  | Compulsory: |  |  |
| 9. |  | Describe the design of batch reactor with neat sketch and list out important components. | CO3 | 20 |