Reg.No. ____________

End Semester Examination – April/May – 2018

| # Code : | # 14CE2001 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # SURVEY | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. |  | The following consecutive readings were taken with a level and a 4m staff on a continuously sloping ground at a common interval of 30m 0.780,1.535,1.955,2.430,2.985,3.480,1.155,1.960,2.365,3.640,0.935,1.045,1.630,2.545. The Reduced level of first point was 180.750 m. Calculate the reduced level by rise and fall method. | CO3 | 20 |
| (OR) |  |  |  |  |
| 2. | a. | What is triangulation? Explain the classification of triangulation system | CO3 | 10 |
|  | b. | What do you mean by total station? Write the merits of total station over theodolite | CO1 | 10 |
| 3. |  | For a closed traverse ABCDE the length and bearings of the lines are given as follows Line; Length (m); Bearing. AB; 362.55; N33Ο10’W. BC; 218; N39Ο08’E. CD; 163.22; S10Ο20’E. DA; 195.95; S66Ο50’E. EA; 278.53; S32Ο20’W. Compute the latitudes, departures, closing error. Adjust the latitudes and departure by Bowditch’s rule and then compute the adjusted length of each line | CO3 | 20 |
| (OR) |  |  |  |  |
| 4. |  | The measured lengths and bearings of the sides of a closed traverse ABCDEA run in an anticlockwise direction and are tabulated below Line; Length (m); Bearing. AB; 298.7; 0Ο0’. BC; 205.7; N25Ο12’W. CD; ?; S75Ο6’W. DA; ?; S56Ο24’E. EA; 213.4; N35Ο36’E. Calculate the lengths of CD and DE | CO2 | 20 |
| 5. | a. | Explain Theory and principles of Stadia tacheometry | CO1 | 10 |
|  | b. | In which situation tacheometric surveying can be used. | CO2 | 10 |
| (OR) |  |  |  |  |
| 6. |  | To determine the elevation of station P in a tacheometric survey, the following observations were made with the staff held vertical. The instrument was fitted with an anallactic lens and its multiplying constant is 100. Instrument station; H.I.(m); Staff station; Vertical angle; Staff readings (m). O; 1.45; B.M.; -6Ο00’; 1.335,1.895,2.460. O; 1.45; C.P.; +8Ο30’; 0.780,1.265,1.745. P; 1.40; C.P.; -6Ο30’; 1.155,1.615,2.075. If R.L. of B.M. is 250m, calculate the R.L. of P. | CO3 | 20 |
| 7. | a. | Define the terms point of curve and point of tangency | CO3 | 5 |
|  | b. | Explain reverse curve and where do you recommend it. | CO2 | 5 |
|  | c. | What are the different methods of curve setting | CO3 | 5 |
|  | d. | How a curve is designated? | CO2 | 5 |
| (OR) |  |  |  |  |
| 8. |  | Two straight lines AC and CB to be connected by a 3° curve, intersect at a chainage of 2760 m the deflection angle being 30°. Calculate all the necessary data for setting out the curve by the method of offsets from the long chord. | CO2 | 20 |
|  |  | Compulsory: |  |  |
| 9. | a. | Describe the uses of contour and contour map | CO2 | 5 |
|  | b. | Define Line of collimation and Reduced Level | CO1 | 5 |
|  | c. | Explain the classification of surveying | CO2 | 5 |
|  | d. | Explain the properties of contour | CO3 | 5 |