Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code: | # 14CE2001 | # Duration:3hrs |  |
| --- | --- | --- | --- |
| # Subject: | # SURVEY | # Marks : 100 |  |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome |  | Marks |
| --- | --- | --- | --- | --- | --- |
| 1. |  | The following consecutive readings were taken with a level and a 4m staff on a continuously sloping ground at a common interval of 30m 0.780, 1.535, 1.955, 2.430, 2.985, 3.480, 1.155, 1.960, 2.365, 3.640, 0.935, 1.045, 1.630, 2.545. The Reduced level of first point was 180.750 m. Calculate the reduced level by Height of collimation method. | CO2 |  | 20 |
| (OR) |  |  |  |  |  |
| 2. |  | The following readings were taken a dumpy level: 3.864, 3.346, 2.932, 1.952, 0.854, 3.796, 2.639, 1.542, 1.934, 0.864, and 0.665. The level was shifted after fifth and eighth readings. The first reading was taken on a benchmark of R.L 150.250m. Calculate the reduced levels of the change points, and the difference of level between the first and last points. | CO2 |  | 20 |
| 3. |  | ABCDA is a closed traverse in which the bearing of DA and the length of BC have not been recorded. The rest of the field notes are as follows Line; Length (m); Bearing. AB; 335; 181⁰18’. BC; ?; 90⁰0’. CD; 408; 357⁰36’. DA; 828; ?. Find the missing data | CO3 |  | 20 |
| (OR) |  |  |  |  |  |
| 4. |  | For a closed traverse ABCDE the length and bearings of the lines are given as follows: Line; Length (m); Bearing. AB; 362.55; N33010’W. BC; 218; N39008’E. CD; 163.22; S10020’E. DA; 195.95; S66050’E. EA; 278.53; S32020’W. Compute the latitudes, departures, closing error. Adjust the latitudes and departure by Bowditch’s rule and then compute the adjusted length of each line. | CO3 |  | 20 |
| 5. |  | Determine the horizontal distance and reduced level of a staff station by stadia tacheometry from the following data: the instrument is at point P; the sighting is to point A; the mean vertical angle is 10041'30" elevation and the staff readings in meters are 0.915, 1.435 and 1.955 the stadia constants are 99.5 and 0.2m the RL of the instrument station = 35.42m and height of instrument axis is 1.52m. | CO3 |  | 20 |
| (OR) |  |  |  |  |  |
| 6. |  | To determine the elevation of station P in a tacheometric survey, the following observations were made with the staff held vertical. The instrument was fitted with an anallactic lens and its multiplying constant is 100. Instrument station; H.I.(m); Staff station; Vertical angle; Staff readings (m). O; 1.45; B.M.; -6000’; 1.335,1.895,2.460. O; 1.45; C.P.; +8030’; 0.780,1.265,1.745. P; 1.40; C.P.; -6030’; 1.155,1.615,2.075. If R.L. of B.M. is 250m, calculate the R.L. of P | CO3 | 20 |
| 7. |  | A circular curve has 300 m radius and 600 deflection angle. What is its degree by (a) arc definition and (b)chord definition of standard length 30 m. Also calculate (i) length of curve, (ii) tangent length, (iii) length of long chord, (iv)mid-ordinate and (v)apex distance. |  | CO1 | 20 |
| (OR) |  |  |  |  |  |
| 8. | a. | List the types of curve with neat sketch. | CO1 |  | 10 |
|  | b. | Define Transition curve. What are the functions of transition curve. | CO1 |  | 10 |
|  |  | Compulsory: |  |  |  |
| 9. | a. | What do you mean by total station? Write the merits of total station over theodolite. | CO2 |  | 10 |
|  | b. | What is triangulation? Explain the classification of triangulation system. | CO3 |  | 10 |