Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 14CE2002 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | MECHANICS OF SOLIDS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. |  | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | An axial pull of 40kN is acting on a bar consisting of three sections of length 300mm, 250mm and 200mm and of diameters 20mm, 40mm and 50mm respectively. Find the stresses in each section and total extension of the bar. Take E=2.1*105 N/mm2. | CO1 | 8 |
|  | b. | A steel rod of 20mm diameter is enclosed centrally in a hollow copper tube of external diameter 40mm and internal diameter 35mm. The composite bar is then subjected to an axial pull of 50kN. Find the stresses in the rod and the tube. Take Es=2.1*105 N/mm2 and Ec=1*105 N/mm2. | CO1 | 12 |
| (OR) |  |  |  |  |
| 2. | a. | A gun metal rod 25mm diameter screwed at the end passes through a steel tube 30mm and 35mm internal and external diameters. The temperature of the whole assembly is raised to 125oC and the nuts on the rod are then screwed lightly home on the ends of the tube. Calculate the stresses developed in gun metal and steel tube when the temperature of the assembly has fallen to 20oC. Take Young’s Modulus of gun metal and steel as 1*105 N/mm2 and 2.1*105 N/mm2. And co-efficient of temperature of gun metal and steel as 20*10-6/oC and 12*10-6 /oC. | CO1 | 14 |
|  | b. | Determine the change in length, breadth and thickness of a steel bar which is 5m long,20mm wide and 15mm thick subjected to an axial pull of 100kN in the direction of its length. Take E=200GPa and Poisson’s ratio=0.3. | CO1 | 6 |
| 3. |  | At a certain point of a control lever of an aeroplance, there are normal stresses of 75MN/m2 tension and 45MN/m2 compression on two planes at right angles to one another together with shearing stresses of 65MN/m2 on the same plane when it just takes off. After it gathers full speed, it is observed that the loading on this element gets so increased, that the stresses reach value x times those observed at the start. Compute the maximum value of x if the maximum direct stress in the materials not to exceed 115MN/m2 and the maximum shearing stress is not to exceed 120MN/m2. Hence or otherwise, state values of the principal stresses and the maximum shearing stresses actually developed and locate their planes. | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. |  | A rectangular block is subjected to a compressive stress of 150N/mm2 on one plane and accompanied by shear stress of 72N/mm2. Determine the postion of principal planes and maximum and minimum intensity of direct stress. Find also the maximum intensity of shear stress. Comapre the results obtained from Mohr’s circle with the analytical method. | CO2 | 20 |
| 5. |  | A beam AB 10m long has supports at its ends A and B. It carries a point load of 2.5kN at 3m from point A and a point load of 2.5kN at 7m from A and a uniformly distributed load of 0.5kN/m between the point loads. Draw the shear force diagram and bending moment diagram for the beam. Find the loacation of contraflexure point and the value of maximum bending moment. | CO3 | 20 |
| (OR) |  |  |  |  |
| 6. |  | A horizontal bema AB 8m long is supported at A and C, 6m from A. The beam supports a UDL of 1.5kN/m over its entire length and also concentrated loads of 3kN and 1.5kN at D and B respectively, D being 2m long from A. Draw the SF and BM diagrams for the beams. Where does the maximum BM occur and what is its value? | CO3 | 20 |
| 7. |  | The cross-section of a beam is as shown in the figure. The beam is made of a material with permissible stress in compression and tension equal to 1000N/mm2 and 1400N/mm2 respectively. Calculate the moment of resistance of the cross-section, when subjected to a moment causing compression at the top and tension at the bottom. Also calculate the compression in the top flange and the tension in the bottom flange corresponding to this moment. | CO4 | 20 |
| (OR) |  |  |  |  |
| 8. |  | A solid shaft is to tranmsit 300kW at 100 rpm if the shear stress is not to exceed 80N/mm2. Find the diameter of the shaft. If this shaft were to be replaced by hollow shaft os same material and length with an increase in diameter of 0.6 times the external diameter, what percentage saving in weight os possible? | CO4 | 20 |
|  |  | Compulsory: |  |  |
| 9. |  | A cantilever beam of length 7m carries a UDL of 18kN/m over a length of 3m from the free end along with a point load 2kN at3m from the free end. Determine the deflection at free end of the beam. Take E=2.1*105 N/mm2 and I=1.2*108 N/mm4. | CO4 | 20 |