Reg.No. ____________

End Semester Examination – Apr/May – 2018

| Code : | 14CE2002 | Duration : | 3hrs |
| --- | --- | --- | --- |
| Sub. Name : | MECHANICS OF SOLIDS | Max. marks : | 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. |  | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | A member shown in figure is subjected to point loads. Calculate the force P necessary for equilibrium and total elongation of the bar. | CO1 | 8 |
|  | b. | A steel rod 20 mm in diameter passes centrally through a steel tube of 25 mm internal diameter and 30 mm external diameter. The tube is 800 mm long and is closed by rigid washers of negligible thickness which are fastened by nuts threaded on the rod. The nuts are tightened until the compressive load on the tube is 20 kN. Calculate the stresses in the tube and the rod. Find the increase in these stresses when one nut is tightened by one-quarter of a turn relative to the other. There are 4 threads per 10 mm. take E = 2 x 105 N/mm2. | CO1 | 12 |
| (OR) |  |  |  |  |
| 2. | a. | A flat bar of aluminium alloy 25 mm wide and 5 mm thick is placed between two steel bars each 25 mm wide and 10 mm thick to form a composite bar 25 mm x 25 mm. The three bars are fastened together at their ends when the temperature is 15oC. Find the stress in each of the materials when the temperature of the whole assembly is raised to 55oC. If at the new temperature a compressive load of 30 kN is applied to the composite bar what are the final stresses in steel and alloy. Take Es = 200 GN/m2, Eal = Es/3, αs = 1.2 x 10-5 per oC and αal = 2.3 x 10-5 per oC | CO1 | 14 |
|  | b. | A bar of 30 mm diameter is subjected to a pull of 60 kN. The measured extension on gauge length of 200 mm is 0.1 mm and change in diameter is 0.004 mm. Calculate young’s modulus, poisson’s ratio and bulk modulus. | CO1 | 6 |
| 3. |  | An elemental cube is subjected to tensile stresses of 30 N/mm2 and 10 N/mm2 acting on two mutually perpendicular planes and a shear stress of 10 N/mm2 on these planes. Determine the magnitudes and directions of principal stresses and also the maximum shear stress using - Analytical method. - Graphical method (Mohr’s circle). | CO2 | 20 |
| (OR) |  |  |  |  |
| 4. |  | At a point in a material under stress the intensity of stresses on planes AB and BC is shown in figure. Determine the following - The resultant stress on plane BC - The principal planes and their directions and - The maximum shear stresses and their planes. | CO2 | 20 |
| 5. | a. | 1kN 1 kN 2 kN/m B C D E 1m 1m 2m 1m A Draw the S.F and B.M diagram for a cantilever beam as shown in figure. | CO3 | 8 |
|  | b. | Draw the shear force and bending moment diagrams for the beam shown in figure. Also determine the maximum bending moment. F E A B D C 1 m 2 m 2 kN/m 1 m 1 m 1 m 1 kN 4 kN | CO3 | 12 |
| (OR) |  |  |  |  |
| 6. |  | For the beam shown in fig. calculate the value of U.D.L. w kN/m so that ending moment at C is 50 kNm. Draw the S.F and B.M diagrams for this beam and locate the point of contraflexure. B D A C 2 m 4 m 4 m w kN/m 20 kN | CO3 | 20 |
| 7. |  | The cross-section of a cast iron beam is as shown in Figure. The top flange is in compression and bottom flange is in tension. Permissible stress in tension is 30 N/mm2. Find the safe uniformly distributed load the beam can carry over a simply supported span of 5 m. Also find the maximum compressive stress. | CO4 | 20 |
| (OR) |  |  |  |  |
| 8. | a. | A solid phosphor bronze shaft of 80 mm diameter is coupled to a hollow steel shaft 80 mm outside diameter. The torque applied to the compound shaft develops a maximum shear stress of 40 MPa in the bronze shaft and 72 MPa in steel shaft. The length of the steel shaft is 1 m and of bronze shaft is 1.2 m. The angle of twist for the steel shaft is not to exceed 1o. If modulus of rigidity of steel is 80 GN/m2 and of bronze is 40 GN/m2 find: - Internal diameter of the shaft. - Total angle of twist for the whole shaft. - Power transmitted by the shaft if the speed is 10 r.p.s. | CO4 | 14 |
|  | b. | A hollow shaft, having an internal diameter 60% of its external diameter, is to replace a solid shaft transmitting the same power at the same speed. Calculate the percentage saving in material, if the material to be used is also the same. | CO4 | 6 |
|  |  | Compulsory: |  |  |
| 9. |  | A simply supported beam of 6 m span is subjected to two point loads of each 60 kN at 2 m and 4m from the left end. Also it carries a UDL of 10 kN/m on the entire length of the beam. Design the beam as a rectangular section keeping breadth as half of depth. The permissible bending stress for the beam material is 120 N/mm2. Neglect the self weight of the beam. | CO3 | 20 |