Reg.No. ____________

End Semester Examination – Apr/May – 2018

| # Code : | # 14CE2002 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # MECHANICS OF SOLIDS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- |
| 1. | a. | The following data refer to a mild steel specimen tested in laboratory. Diameter of the specimen = 40 mm Length of the specimen = 400 mm Extension under a load of 25kN = 0.05 mm Load at yield point = 135.5kN Maximum load = 240.55kN Length of the specimen after failure = 430 mm Neck diameter = 25.5 mm Determine: - Young’s modulus - Yield point - Ultimate stress - Percentage of elongation - Percentage of reduction in area - Safe stress, adopting a factor of safety of 2 | CO1 | 14 |
|  | b. | Draw the stress- strain diagram of mild steel and mark the major points | CO1 | 6 |
| (OR) |  |  |  |  |
| 2. | a. | A compound tube consists of a steel tube 140 mm internal diameter and 160mm external diameter and an outer brass tube 160 mm internal diameter and 180 mm external diameter . The two tubes are of the same length. The compound tube carries an axial compressive load of 900 kN. Determine the load carried by each tube and the decrease in length. The length of each tube is 140 mm. The Young`s Modulus of steel and brass are 200 GPa and 100GPa respectively. | CO1 | 14 |
|  | b. | Derive the relationship between Elastic modulus (E) and Bulk modulus (K) | CO1 | 06 |
| 3. | a. | The principal stress at a point across two planes are 100 N/mm2 (tensile) and 50 N/mm2(tensile).Determine the normal, tangential stress and the resultant stress on a plane inclined at 20oto the axis of minor stresses | CO1 | 10 |
|  | b. | The principal stresses at a point across two perpendicular planes are 90 N/mm2 and 40 N/mm2. Find the normal stress, tangential stress and resultant stress and its obliquity on a plane at 30o with the major principal plane. Find also the intensity of stress which acting alone can produce the same maximum strain. Take poisons ratio as 0.25 | CO1 | 10 |
| (OR) |  |  |  |  |
| 4. | a. | How do you determine the Principal stresses and maximum shear stress graphically? | CO3 | 5 |
|  | b. | Explain in detail the theories of failure - The maximum principal stress theory - The maximum strain theory - The maximum shear stress theory | CO4 | 15 |
| 5. | a. | Plot the shear force and bending moment diagram for the simply supported beam shown in figure. | CO2 , CO4 | 18 |
|  | b. | Define shear force | CO2 | 2 |
| (OR) |  |  |  |  |
| 6. | a. | A simply supported beam of length 8m rests on supports 6m apart, the right hand end is overhanging by 2m. the beam carries a UDL of 1500N/m over the entire length. Draw the shear force and bending moment diagrams and find the point of contraflexure. | CO2 , CO4 | 15 |
|  | b. | Explain the sign conventions for shear force and bending moment in general. | CO2 | 5 |
| 7. | a. | A timber beam is 120mm wide and 200mm deep and is used on a span of 4metres. The beam carries a UDL of 2.8 kN/m run over the entire length. Find the maximum bending stress induced. | CO3 | 10 |
|  | b. | A flitched beam consists of a wooden joist 15 cm wide and 30 cm deep strengthened by steel plates 1 cm thick and 30 cm deep one on either side of the joist. If modulus of elasticity of steel is 20 times that of wood, then the calculate the width of equivalent wooden section? | CO3 | 10 |
| (OR) |  |  |  |  |
| 8. | a. | A beam of I section is having overall depth as 500mm and overall width as 190mm. The thickness of flanges is 25mm whereas the thickness of the web is 15mm. the moment of inertia about N.A is given as 6.45x108 mm4. If the section carries a shear force of 40 kN , calculate the maximum shear stress. Also calculate the shear stress distribution across the section. | CO3 | 15 |
|  | b. | Show that for a rectangular section of the maximum shear stress is 1.5 times the average shear stress. | CO4 | 5 |
|  |  | Compulsory: |  |  |
| 9. | a. | A solid steel shaft has to transmit 100kW at 250 r.p.m. Taking allowable shear stress as 70 N/mm2, find the suitable diameter for the shaft, if he maximum torque transmitted at each revolution exceeds the mean by 20% | CO3 | 14 |
|  | b. | List out the assumptions made in the derivation of shear stress produced in a circular shaft subjected to torsion | CO4 | 6 |