Reg.No. ____________

End Semester Examination – April/May 2018

| # Code : | # 14CE2002 | # Duration : | # 3hrs |
| --- | --- | --- | --- |
| # Sub. Name : | # MECHANICS OF SOLIDS | # Max. marks : | # 100 |

ANSWER ALL QUESTIONS (5 x 20 = 100 Marks)

| Q. No. | Sub Div. |  | Questions | Course Outcome | Marks |
| --- | --- | --- | --- | --- | --- |
| 1. | a. |  | Draw the stress- strain diagram of mild steel. Explain salient points. | CO1 | 4 |
|  | b. |  | Determine the values of Young’s modulus and Poisson’s ratio of a metallic bar of length 25 cm, breadth 5 cm and depth 5 cm when the bar is subjected to an axial compressive load of 1.75 MN. The decrease in length is 0.175 cm and increase in breadth is 0.013 cm. | CO4 | 4 |
|  | c. |  | In a tension test on a mild steel bar of 30 mm diameter and 300 mm gauge length, the following observations were made Extension under a load of 20kN = 0.045 mm Load at yield point = 127.65 kN Maximum load = 210.55 kN Length of the specimen after failure = 390 mm Neck diameter = 18.25 mm Determine: - Young’s modulus - Yield point - Ultimate stress - Percentage of elongation - Percentage of reduction in area - Safe stress, adopting a factor of safety of 1.5 | CO2 | 12 |
| (OR) |  |  |  |  |  |
| 2. | a. | A load of 270 kN is applied on a short concrete column 250 mm x 250mm. The column is reinforced with 8 bars of 16 mm diameter. If the modulus of elasticity of steel is 18 times that of concrete, find the stresses in concrete and steel. If the stresses in concrete shall not exceed 4 N/mm2, evaluate the area of steel required so that the column can support a load of 500 kN. |  | CO2 | 10 |
|  | b. | Explain the relevant points of stress- strain diagram of mild steel with the help of a neat sketch. |  | CO4 | 4 |
|  | c. | A bar of 40 mm diameter is subjected to a pull of 50 kN. The measured extension on gauge length of 200 mm is 0.2 mm and change in diameter is 0.006mm. Calculate (i) Young’s modulus (ii) Poisson’s ratio (iii) Bulk modulus |  | CO2 | 6 |
| 3. | a. | Draw the shear force and bending moment diagrams of a cantilever beam carrying udl of ‘w’ per meter throughout its span of ‘L’ meter. |  | CO2,CO3 | 4 |
|  | b. | Derive the general relationship between shear force and bending moment at any section of a beam. |  | CO3 | 4 |
|  | c. | Compute the reactions of supports, shear force and bending moments at different points. Sketch the shear force and bending moment diagrams. |  | CO3 | 12 |
| (OR) |  |  |  |  |  |
| 4. | a. |  | Name the types of beams based on support conditions. | CO4 | 2 |
|  | b. |  | Define the term “point of contraflexure”. | CO4 | 2 |
|  | c. |  | Draw shear force and bending moment diagrams for a cantilever beam with point load (W) at the free end, the length of the beam is ‘L’. | CO3 | 4 |
|  | d. |  | Calculate the support reactions. Portray the bending moment and shear force diagrams. Determine the point of contraflexure within the span AB and show their positions in the bending moment diagram. | CO3 | 12 |
| 5. | a. |  | Give any four assumptions in theory of simple bending. | CO4 | 4 |
|  | b. |  | A leaf spring is made up of 7 steel plates, 65mm wide and 6.3mm thick. Determine length of spring so that it may carry a central load of 2750N, stress being limited to 160 N/mm2. | CO2,CO3 | 8 |
|  | c. |  | A cantilever beam of cross section 100 mm wide and 200 mm deep carries a point load of 10 kN at the free end of the beam. The bending stress is not to exceed 50 N/mm2. Evaluate the span of the cantilever beam. | CO2,CO3 | 8 |
| (OR) |  |  |  |  |  |
| 6. | a. |  | Define Section modulus. Estimate the section modulus value of a rectangular section with depth ‘d’ and width ‘b’. | CO2 | 4 |
|  | b. |  | A cantilever beam of length 4 m has the cross section of ‘I’ shape with unequal flanges. The widths of top and bottom flanges are 250 mm and 150 mm respectively. Thickness of the section is 50 mm uniform throughout. Total depth of the section is 350 mm. The beam carries udl of 12 kN/m throughout its entire span. Sketch the shear stress and bending stress distribution diagrams. | CO2,CO3 | 16 |
| 7. | a. |  | State any four assumptions in the Theory of Torsion. | CO4 | 4 |
|  | b. |  | A solid circular shaft transmits power of 80 kW at a speed of 200 r.p.m. Determine the diameter of the shaft if the twist in the shaft is not to exceed 10 in 2 m length of the shaft and shear stress is limited to 50 MPa. Assume Modulus of Rigidity G = 1 x 105 N /mm2. | CO2,CO3 | 16 |
| (OR) |  |  |  |  |  |
| 8. | a. |  | A close coiled helical spring is made out of 10 mm diameter steel rod. The spring consists of 10 complete turns with a mean diameter of 120 mm. The spring carries an axial pull of 200 N. The modulus of rigidity of steel C = 8 x 10 4 N/mm2. Determine the deflection, maximum shear stress induced in the spring. Also evaluate the stiffness and energy stored in the spring. | CO2,CO3 | 12 |
|  | b. |  | Differentiate between section modulus and polar modulus. Derive the values of section modulus and polar modulus of a solid circular section having diameter ‘d’. | CO4 | 8 |
|  |  |  | Compulsory: |  |  |
| 9. |  |  | A simply supported beam of I – section having a top flange of 30 mm x 5 mm, web of 35 mm x 5 mm and the bottom flange of 30 mm x 5 mm spans two supports 1.5m apart. Determine the total load uniformly distributed on the entire span that the beam could carry in addition to a concentrated load of 8 kN at 1 m from left end in order that extreme fiber stress is limited to 800 N/mm2. | CO3 | 20 |